3GPP TSG-WG SA2 Meeting #171	S2-2509018

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

# Title:	[DRAFT] Reply LS on compatibility issue for PEI and emergency PDU session

# Response to:	S2-2508189/R3-255906: LS on compatibility issue for PEI and emergency PDU session

# Release:	Release 19

# Work Item:	NR_UE_pow_sav_enh-Core, NR_LPWUS-Core

Source:	[Huawei to be] SA2

To:	RAN3, CT1

Cc:	-

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT Wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR6457, TS 23.502 CR5594.

1. Overall Description:

SA2 would like to thank RAN3 on the LS on compatibility issue for PEI and emergency PDU session (S2-2508189/R3-255906).

Taking RAN3’s input as the starting point, SA2 has discussed the proposed compatibility issues and the corresponding solution, reached a consensus on the solution, and regarding the questions in the LS, has the following answers:

Q1: Whether it is possible for SA2 to revert the R19 agreed CRs on removing the restriction of emergency PDU session for PEI?

Answer: No.

Q2: Whether it is possible for SA2 to remove the restriction of emergency PDU session for PEI from R17 onwards?

Answer: No.

Q3: An unified solution to be considered to address the compatibility issue solely through modifications to the Rel-19 RAN or/and CN specification?

Answer: Yes, the agreed solution is in the attached CRs.

Q4: R17/R18 gNBs not use the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19)?

Answer: SA2 only specifies that R17/R18 gNBs shall not use the CN based subgrouping (rather than the complete PEI) in the presence of emergency PDU session.

Q5: R17/R18 UEs ignore the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19)?

Answer: SA2 only specifies that R17/R18 UEs shall not use the CN based subgrouping (rather than the complete PEI) in the presence of emergency PDU session.

Q6: R17/R18 UEs initiate a registration procedure without indicating its capability of supporting PEIPS after the emergency PDU session is set up, so that AMF will not use the PEIPS?

Answer: Yes, AMF will not use the PEIPS and will not provide the Negotiated PEIPS assistance information to the UE in the registration accept message.

2. Actions:

To RAN3, CT1 groups.

ACTION: 	SA2 asks RAN3 and CT1 to take the above answers and attached CRs into account and update their specifications as necessary.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US

TSG-SA2 Meeting #173		09th Feb – 13th Feb, 2026		India, IN