3GPP TSG-WG SA2 Meeting #171	S2-2509045

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#7, NTN] NTN specific WTs and KIs for 6G support of NTN

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This contribution proposes the scope of WT#7 and corresponding Key Issue(s) for SA2 6G SID.

# 1.	Introduction

During SA2#170 S2-2508134 was marked as postponed and represented the latest version of the WT & KI proposals after discussions in the meeting.

The contents of S2-2508134 are used as the basis of this update, which clarifies several aspects.

# 2.	Justifications

## 2.1	General

The 6G SID in SP-2506096 contains the following WT for NTN:

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 8: The detailed scope for WT#7 will be coordinated and aligned with RAN

This discussion on the justification on WT#7 looks at architectural motivations for included aspects under this WT, along with gaps there are with other WTs and 5G and the technical impacts from that.

## 2.2	Architectural Motivation

6G NTN should become in general part of 6G and treated like the other 6G RATs. Therefore, in general, this WT should only consider aspects which are specific to NTN scenarios.

Many of the aspects covered in other WTs will by default apply to NTN, and while tempting to attempt to list those aspects, they will become “business as usual”, and therefore they do not need listing or studying under the NTN WT.

This WT should only consider aspects which are specific to NTN, and not aspects common to NTN and TN.

## 2.3	Multi-Orbit Considerations

The Proposal in S2-2508134 contained the following NOTE:

NOTE 2: The gap due to multi-orbit and frequent TN-NTN/NTN-NTN mobility scenario can be studied based on the agreed TN mobility mechanism for 6G.

The text about frequent mobility scenarios between TN and NTN, or NTN and NTN is not supported by any SA1 requirements related to multi-orbit.

In general, as 6G NTN is the same RAT as 6G TN, there is no special requirements for mobility between TN and NTN, and general mobility is covered by other WTs.

Therefore. this aspect should be removed.

## 2.4	Backhaul Impacts

There was discussion at SA2#170 about impacts that backhaul can have, and whether mitigations should be studied and the following EN was suggested:

Editor's Note: Whether the improvement of backhaul via 6G satellite access is to be considered is FFS.

During the satellite studies in 5G, the satellite backhaul connecting RAN and CN is via feeder links and inter-satellite links in two cases: (1) with RAN and CN located on the ground (also known as the reflective architecture), and (2) with RAN on board and CN on the ground or possibly on other satellites (also known as the regenerative architecture).

With RAN on board communicating with CN nodes on the ground then the backhaul from RAN towards ground will use feeder links, and with RAN on board communicating and CN nodes on other satellites then inter-satellite links will be used.

Satellite backhaul via feeder links and/or inter-satellite links has the following characteristics:

-	The Feeder links can switch quickly due to fast movement of the satellites.

-	The inter-satellite links are significantly less stable when compared with e.g., fibre links, especially for cross-orbit inter-satellite links, due to harsh environment and relative movement of satellites.

The above characteristics may cause problems including:

-	High jitter, high packet loss rate, and packet disordering, which in turn causes the drop of TCP throughput. According to “LEO Satellite vs. Cellular Networks: Exploring the Potential for Synergistic Integration” [1], TCP severely suffers from such a high packet loss of Starlink, leading to only 1/5 of the throughput achieved by UDP over Starlink.

-	Additionally, it leads to long packet delay due to retransmission and slow start of TCP.

The assumption of high reliability in terrestrial transport networks does not apply in the NTN scenario. NTNs need to address challenges such as higher packet loss, higher jitter, and higher packet reordering to prevent degradation of user experience and maintain system performance.

Considering the above observations and considerations, it is can be seen that NTN backhaul presents challenges compared to TNs, and this is specific to NTN. Therefore it should be studied.

# 3. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *(All new)

## Annex A.X 	WT#7 Scope

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN.

NOTE 2:	The gap due to multi-orbit scenario can be studied based on the agreed TN mobility mechanism for 6G.

1. -	Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

Editor's Note:	Whether any QoS/policy improvement is needed for satellite backhaul is FFS.

# * * * * Second change * * * *(All new)

## 5.X	Key Issue #X: Support 6G RAT for NTN

To support 6G RAT for NTN, this key issue studies the following issues:

NOTE 1:	This key issue will be coordinated and aligned with RAN.

NOTE 2:	The gap due to multi-orbit scenario can be studied based on the agreed TN mobility mechanism for 6G.

1. -	Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

Editor's Note:	Whether any QoS/policy improvement is needed for satellite backhaul is FFS.

# * * * * End of changes * * * *

# 4. References

[1] Bin Hu, Xumiao Zhang, Qixin Zhang, Nitin Varyani, Z. Morley Mao, Feng Qian, and Zhi-Li Zhang. 2023. LEO Satellite vs. Cellular Networks: Exploring the Potential for Synergistic Integration. In Companion of the 19th International Conference on Emerging Networking EXperiments and Technologies (Paris, France) (CoNEXT 2023). Association for Computing Machinery, New York, NY, USA, 45–51. doi:10.1145/3624354.3630588.