3GPP TSG-SA WG2#171	S2-2509068

13 – 17 October, 2025, Wuhan, China	(Revision of S2-2508135, 8026)

Source:	[Sony, Qualcomm, Interdigital, CATT, MediaTek Inc, Nokia, Vivo,] Xiaomi, [NTT DoCoMo]

Title:	[WT#8] Scope on 6G IoT

Document for:	Approval

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This contribution proposes a scope description for WT#8 on 6G IoT.

1.	Discussion

2.	Text proposal

In the merger of all the above documents, the “User Name” (company + tdoc number) shows from which document each of the specific text in the WT objective description comes from.

It is proposed to agree the following changes to TR 23.801-01:

# >>>>FIRST CHANGE<<<<

A.8	WT#8: Study whether and how to support cellular IoT enablers in 6G

Study whether and how to support cellular IoT enablers in 6G, including:

- Analyze which 5GS IoT features and solution can be treated as the start point for the study discussion.

- Potential enhancement in 6G to the selected 5GS IoT features and solutions are not precluded.

- Simplify/enhance the Power Consumption (e.g. power saving mechanism) of IoT device.

- Support lightweight procedure to deliver data for the diverse IoT device types efficiently.

Features driven by this WT are expected to be generic and can apply to any UE in 6G.

NOTE y:	The detailed scope for WT#8 will be coordinated and aligned with RAN. Ambient IoT, NB-IoT and non-3GPP access are not in the scope of the study.

# >>>>SECOND CHANGE<<<< (all new text)

5.X. Key Issue #8: Study how to support cellular IoT enablers in 6G

The 5GS supports the Control Plane CIoT 5GS Optimisations and User Plane CIoT 5GS Optimisations over E-UTRA as described in TS 23.501 [2]. The Cellular IoT is in earlier 3GPP releases also referred to as Machine Type Communication (MTC) (see clause 4.3.17 of TS 23.401 [x]).

Though motivated by scenarios and use cases defined in TS 22.261 [y], the functions added to support CIoT have general applicability and are in no way constrained to any specific scenario, use case or UE types, except where explicitly stated. In the context of CIoT the term AF denotes an SCS/AS as defined TS 23.682 [z].

While 5G has made significant strides for CIoT, challenges remain including the CIoT 5GS optimisations is only supported over E-UTRA, CIoT 5GS optimisations are not supported over Non-3GPP RAT type accesses, and the new services (e.g., AI, sensing) supporting for the CIoT is FFS. TR 22.870 [w] has several requirements for the 6G CIoT. This key issue will study how to support cellular IoT enablers in 6G.

In particular, this key issue will study whether and how to support cellular IoT enablers in 6G, including:

- Identify which 5GS IoT features and solutions can be treated as the start point for the study discussion, and how to enhance them for the 6G IoT.

- Simplify/enhance the Power Consumption (e.g. power saving mechanism) of IoT device.

- Support lightweight procedure to deliver data for the diverse IoT device types efficiently.

# >>>>THIRD CHANGE<<<<

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[x]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[y]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[z]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[w]	3GPP TS 22.870: " Study on 6G Use Cases and Service Requirements; Stage 1".

# >>>>END OF CHANGES<<<<