SA WG2 Meeting #171	S2-2509070

13 – 17 October, 2025, Wuhan, China	 (Revision of S2-2508136, 8040, 7974, 7887, 6522)

Source:	[InterDigital Inc., LG Electronics (?), vivo (?), Apple (?), CEWiT (?), IITB (?),Turkcell (?), Nokia, Ericsson (?), T-Mobile USA (?), CATT (?),] Xiaomi, [Huawei (?), NEC]

Title:	[WT#1.2, Network Exposure Framework] Network Exposure Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides more details about what can be studied within Work Task #1.2 on the topic of Network Exposure Framework.

1. Source Documents

This paper is a merge of the following documents.

| AI | TD# | Type | Doc For | Subject | Source | Rel | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Network Exposure |  |  |  |  |  |  |  |
| 20.6.1.2 | S2-2506522 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure Framework] Network Exposure Framework | InterDigital Inc. | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506625 | P-CR | Approval | 23.801-01: [WT#1.2, Exposure Framework] Scope and New Key Issue for Exposure framework | LG Electronics | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506640 | P-CR | Approval | 23.801-01: [WT#1.2, Network exposure] Support for network exposure to UE in 6G network | vivo | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506802 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure] Scope and KIs definition for Network Exposure in 6G | CEWiT | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506863 | P-CR | Approval | 23.801-01: [WT#1.2] Proposal on a common network exposure framework with in-band information exposure support | Nokia | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506890 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure Framework] Key issue for 6G network exposure | Apple | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2506933 | P-CR | Approval | 23.801-01: [WT#1.2, Network Exposure] Exposure Framework architecture in 6G | Ericsson, T-Mobile USA | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507108 | P-CR | Approval | 23.801-01: Intelligent network exposure | TURKCELL | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507195 | P-CR | Approval | 23.801-01: WT#1.2, 6G Exposure Framework and study proposal | CATT | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507363 | P-CR | Approval | 23.801-01: [WT#1.2, network exposure] WTs and KIs for network exposure framework | Xiaomi | Rel-20 | FS_6G_ARC |
| 20.6.6 | S2-2506995 | P-CR | Approval | 23.801-01: [WT#6] Computing Enabler service, coordination of communication and computing service | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.1.2 | S2-2507305 | P-CR | Approval | 23.801-01: [WT#1.2, QoS Framework] QoS Framework Enhancements | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| 20.6.3 | S2-2507223 | P-CR | Approval | 23.801-01: [WT#3, Network AI Agent] Agentic core network for 6G | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |

**** First Change – All New Text ****

Annex A.X. WT#1.2 Network Exposure Framework

One aspect of this work task is to study whether and how to support and/or enhance the network exposure framework in 6G. Aspects that can be studied include:

- X.1 How to support a common framework for the exposure of network capabilities.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

NOTE 1:	Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters.

  - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

NOTE 2: 	The scope of this work task is to specify the common exposure framework, including possible impact to NEF. This work task will not address the information being exposed defined by other work tasks. Other work tasks can cover services that have an exposure component (e.g. computing and sensing). The exposure framework that is developed in this work task should be adaptable to and suitable for those other services (e.g. computing and sensing).

NOTE 3:	What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks.

- X.2 How to support network capabilities exposure to UE.

  - Whether and how network capabilities should be exposed to a consumer on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context.

  - How to easily authorize consumers of network capabilities when the consumer is resident on the UE.

  - How to support and authorize the UE request for network information exposure.

- X.3 How to support In-band Information Exposure.

  - Whether and how to expose information in-band between application endpoints and 6G network in an operator-controlled manner.

  - Whether and how the exposure/reporting path of an exposed capability can be optimized (e.g. in-band).

  - How to discover, authorize and activate in-band exposure services offered by the network and whether and how to integrate in-band exposure in the common exposure framework.

NOTE 4:	In-band information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 5:	This work task will cover how capabilities are exposed in-band. What capabilities are exposed in-band will be coordinated with the work that is done under the User Plane and QoS aspects of Work Task 1.2.

- X.4 Considerations for Intent-Based Exposure.

  - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure.

The scenarios for intent based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces can achieve any intents that are identified within work task #3

NOTE 6:	Information exposure is based on operator policies/SLAs.

NOTE 7:	The goal of intent-base exposure is to make accessing exposed services more user friendly.

NOTE 8:	This aspect of work task has dependency on Work Task #3. The scenarios for intent based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces can achieve any intents that are identified within work task #3. In terms of supporting intent, this work task will consider what functionality is need on the southbound interfaces to support intent based northbound APIs.

NOTE 9:	This work task requires coordination with SA3 (e.g. user consent, privacy and anonymization), SA6 (e.g. CAPIF), and SA5 (e.g. OAM Services).

NOTE 10:	This work task has dependency on service requirements to be specified by SA WG1.

Editor's note:	Any additional update of WT#1.2 network exposure framework, according to the NWM discussion, will be included here.

**** Second Change – All New Text ****

5.X. Key Issue #1.2.x: Study how to support and/or enhance the Network Exposure in 6G

The 5GS supports external exposure of capabilities of network functions and network information via Network Exposure Function (NEF) as described in TS 23.501[2]. The external exposure of capabilities can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, Analytics reporting capability and Member UE selection capability. The exposure information includes the event-based notifications and reports for NSACF and energy related information etc.

The NEF may support CAPIF functions for external exposure. When CAPIF is supported, an NEF that is used for external exposure supports the CAPIF API provider domain functions including the API exposing function, the API publishing function and the API management function, as described in TS 23.222 [x].

The 5GS architecture allows user plane information exposure to some NFs (e.g., NWDAF/DCCF/MFAF, NEF/AF, SMF and TSNAF/TSCTSF) via service-based interface in UPF.

The 5GS also support the Service Exposure for EPC-5GC Interworking. If the UE is capable of mobility between EPS and 5GS, the network is expected to associate the UE with an SCEF+NEF node for Service Capability Exposure.

While 5G has made significant strides for the network exposure, challenges remain including the capability and information exposure for the new services (e.g., AI, sensing, immersive communications), the efficient way to expose existing services and system requirements etc.

TR 22.870 [y] has several requirements for efficient network exposure. These requirements apply to the capability and information of new services and exist service between the network and the Application.

This key issue will study how to support and/or enhance the network exposure for 6G. In particular, this key issue will study:

- X.1 How to support a common framework for the exposure of network capabilities.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

NOTE 1:	Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters.

  - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

NOTE 2: 	The scope of this work task is to specify the common exposure framework, including possible impact to NEF. This work task will not address the information being exposed defined by other work tasks. Other work tasks can cover services that have an exposure component (e.g. computing and sensing). The exposure framework that is developed in this work task should be adaptable to and suitable for those other services (e.g. computing and sensing).

NOTE 3:	What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks.

- X.2 How to support network capabilities exposure to UE.

  - Whether and how network capabilities should be exposed to a consumer on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context.

  - How to easily authorize consumers of network capabilities when the consumer is resident on the UE.

  - How to support and authorize the UE request for network information exposure.

- X.3 How to support In-band Information Exposure.

  - Whether and how to expose information in-band between application endpoints and 6G network in an operator-controlled manner.

  - Whether and how the exposure/reporting path of an exposed capability can be optimized (e.g. in-band).

  - How to discover, authorize and activate in-band exposure services offered by the network and whether and how to integrate in-band exposure in the common exposure framework.

NOTE 4:	In-band information exposure will not replace existing NEF APIs, e.g., for parameter provisioning.

NOTE 5:	This work task will cover how capabilities are exposed in-band. What capabilities are exposed in-band will be coordinated with the work that is done under the User Plane and QoS aspects of Work Task 1.2.

- X.4 Considerations for Intent-Based Exposure.

  - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure.

NOTE 6:	Information exposure is based on operator policies/SLAs.

NOTE 7:	The goal of intent-base exposure is to make accessing exposed services more user friendly.

NOTE 8:	This aspect of work task has dependency on Work Task #3. The scenarios for intent based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces can achieve any intents that are identified within work task #3. In terms of supporting intent, this work task will consider what functionality is need on the southbound interfaces to support intent based northbound APIs.

NOTE 9:	This work task requires coordination with SA3 (e.g., user consent, privacy and anonymization), SA6 (e.g. CAPIF), and SA5 (e.g. OAM Services).

NOTE 10:	This work task has dependency on service requirements to be specified by SA WG1.

Editor's note:	Any additional update of WT#1.2 network exposure framework, according to the NWM discussion, will be included here.

**** Third Change ****

2	References

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[x]	3GPP TS 23.222: " Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[y]	3GPP TS 22.870: " Study on 6G Use Cases and Service Requirements; Stage 1".

**** End of Changes ****