SA WG2 Meeting #171	S2-2509093

13 – 17 October 2025, Wuhan , China

Source:	Apple

Title:	[WT#6, Computing]  WT update & KI proposal for 6G Computing

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes WT update for 6G Computing and a key issue for coordination of compute and communication resources.

# 1. Justifications

The work task description for WT#6 , contains the sub-WT#6.2 which speaks about coordination of communication and computing to offer service continuity and service experience.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects: - Whether and how to coordinate communication and computing resource(e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF). - Whether and how to identify and/or collect the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.). - Whether and how to expose the computing resource information and/or network metrics. NOTE 2: Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2. - Whether and how to improve the service experience/QoS(e.g., by defining new metrics or enhancing the existing metrics for QoS). - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

Enabling inter-domain coordination between compute and communication is a key aspect for guaranteeing the service experience for new type of applications. 6G architecture should consider the possible deployment scenarios and the possible roles in the coordination that a communication service provider and an application service provider can play.

The support for computing in 6G has to consider the following deployment scenarios: compute sites are hosted in a service hosting environment of the MNO, compute sires are hosted external to 6G CN by a 3rd party.

The coordination between these domains has to consider the following deployment scenarios: coordination performed by an entity in the communication service provider assisted by a 3rd party application service provider,  coordination performed by an external application service provider assisted by the communication service provider.

The key aspects for the study would be to identify the information required for enabling inter-domain coordination, how this information is exposed/provisioned and how is it used.

The inter-domain coordination should also focus on aspects related to service continuity upon change of computing site or access paths.

Following changes are proposed for the work task description and a key issue is proposed based on the sub-WT#6.2.

**** First Change ****

# Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

In order to support computing, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource(e.g., within the core network or outside the core network,  by the operator or a 3rd party owned AF).

  - Whether and how to identify, collect or provision the computing resource related information(e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to improve the service experience/QoS .

  - Whether and how to support service continuity for computing service upon change of computing site, user plane function or access paths (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point) to enable discovery or (re-)selection of computing site/Application Server providing services to UE/AF.

NOTE 3:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

**** Next Change ****

# 5.X. Key Issue #X: Coordination of computing and communication for 6G

A key aspect for the 6G compute framework is to consider compute and communication requirements together. The 6G architecture has to support the coordination between compute and communication domains to enable better service experience.

The support for computing in 6G has to consider the following deployment scenarios: compute sites are hosted in a service hosting environment of the MNO, compute sires are hosted external to 6G CN by a 3rd party.

The coordination between these domains has to consider the following deployment scenarios: coordination performed by an entity in the communication service provider assisted by a 3rd party application service provider,  coordination performed by an external application service provider assisted by the communication service provider.

This key issue aims to study the following:

- X.1: identification of compute and communication related information required for a co-ordinated handling

- X.2: exposure of information across domains to enable the coordination and use of such information to improve service experience/QoS

- X.4: how to enable service continuity for computing upon change of computing site and/or user plane function and access paths through the coordinated handling of compute and communication

NOTE: This key issue requires potential alignment with SA5 (hosting compute resources in a service hosting environment) / SA6 (application layer aspects).

NOTE 2: This key issue requires potential synchronisation with key issues related to work task 1.2.x on exposure, 1.2.x on QoS enhancement.

NOTE 3: The work in this key issue may be influenced by the 6GS architecture definition for WT#3.1 AI for Network, WT#4 Sensing.

**** End of Changes ****