3GPP TSG-SA WG2 Meeting #171	S2-2509156

Wuhan, China, Oct 13 – 17, 2025

Source:	Ericsson

Title:	KI#4, New Sol: UE Geolocation in SIP INVITE for PSAP selection

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: this paper proposes a new solution to address the PSAP selection for IMS emergency over NB-IoT GEO satellite.

# 1. Introduction

This contribution proposes a new solution to Key Issue #4, focus on location service with regards to PSAP selection for IMS emergency.

As described in 23.167, if the UE has location information available, it shall include it in the request to establish emergency session.

4.3.1 General Location Information Principles

The following general principles shall apply regarding the handling of location information:

- If the UE has location information available, the UE shall include the location information in the request to establish an emergency session. The location information may consist of network location information, that is the Location Identifier and/or the Geographical location information

Considering GNSS (Global Navigation Satellite System) capability is mandatory for UE to access NTN (see 36.300 clause 23.21.1), the UE accessing GEO NB-IoT access has geographical location information available.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification

Also, according to GSMA IR.92 clause 5.2.3, and 3GPP TS 24.229 clause 5.1.6.8.2/5.1.6.8.3, the UE must convey its location if available, using Geolocation header and PIDF-LO (Presence Information Data Format Location Object) in SIP INVITE.

5.2.3 UE location management

When initiating an IMS emergency session, the UE must conform to the requirements to convey its location, using the "Geolocation" header field and the PIDF location object in the initial SIP INVITE request, as specified in section 5.1.6.8.2 and section 5.1.6.8.3 of 3GPP TS 24.229 [5].

Based on the above discussion, it is proposed to reuse the Geolocation/PIDF-LO in SIP INVITE for PSAP selection in case of GEO NB-IoT access.

# 2. Proposal

It is proposed to agree the following new solution to 3GPP TR 23.700-19.

* * * Start of change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[21]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[22]	3GPP TS 23.003: "Numbering, addressing and identification".

[23]	3GPP TR 22.887: "Study on satellite access - Phase 4".

[24]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[x]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[y]	GSMA IR.92: "IMS Profile for Voice and SMS".

[z]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

* * * Next change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #1 | X | X |  |  |  |
| #2 | X |  |  |  |  |
| #3 | X |  |  |  |  |
| #4 | X | X | X | X |  |
| #5 | X |  |  |  |  |
| #6 | X |  |  |  |  |
| #7 | X | X |  |  |  |
| #8 | X |  |  |  |  |
| #9 | X |  |  |  |  |
| #10 | X |  |  |  |  |
| #11 | X |  |  |  |  |
| #12 |  | X |  |  |  |
| #13 |  | X |  |  |  |
| #14 |  | X |  |  |  |
| #15 |  | X |  |  |  |
| #16 |  | X |  |  |  |
| #17 |  | X |  |  |  |
| #18 |  | X |  |  |  |
| #19 |  | X |  |  |  |
| #20 |  | X |  |  |  |
| #X |  |  |  | X |  |

* * * Next change all new* * * *

## 6.X	Solution #X: UE Geolocation in SIP INVITE for PSAP selection

### 6.X.0	High-level solution Principles

This solution proposes to re-use the Geolocation/PIDF-LO (Presence Information Data Format Location Object) in SIP INVITE for routing to the correct PSAP in case of IMS emergency services over NB-IoT GEO satellite access.

### 6.X.1	Description

To access NTN, GNSS (Global Navigation Satellite System) capability is required per TS 36.300 [z]. So that, the UE can provide its geographical location in the SIP INVITE message when accessing NB-IoT GEO satellite, where the footpint of the cell may be very large compared to a typical TN cell. Such geographical location information provided by UE can be used for LRF/GMLC to select the routing information to the proper PSAP.

### 6.X.2	Procedures

Figure 6.X.2.1-1 PSAP selection based UE geolocation

- The UE performs emergency attach or requests to establish emergency PDN connection.

- When MME detects the emergency service, it may trigger network induced location report to LRF/GMLC as described in TS 23.271 [x] clause 9.1.17. The ECGI (E-UTRAN Cell Global Identifier) is provided to LRF/GMLC.

- The UE is GNSS capable and has its location information available. The UE includes its Geolocation/PIDF-LO in the SIP INVITE message, as described GSMA IR.92 [y] clause 5.2.3, and TS 24.229 [16] clause 5.1.6.8.2/5.1.6.8.3.

- IMS CN passes the SIP INVITE including Geolocation/PIDF-LO to LRF/GMLC.

- The LRF/GMLC provides the PSAP routing address to IMS CN, based on the UE’s geographical location and possibly other location information (e.g. ECGI reported by MME).

- The emergency call setup continues towards the PSAP.

### 6.X.3	Impacts on Services, Entities and Interfaces

No impact to existing specifications.

* * * End of change * * * *