SA WG2 Meeting #171	S2-2509159

13 – 17 OCT, 2025, Wuhan, China	(Revision of S2-2508129)

Source:	Ofinno

Title:	[WT#2 Migration and Interworking] Scope and key issue on Migration and Interworking

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution updates the work task scope and proposes the key issue for WT#2.

# 0. Background

At SA2 #170, SA2 discussed proposals on updating WTs for FS_6G_ARC. S2-2508129 related to Migration and Interworking as baseline documents and finally were postponed.

Following are text proposals in S2-2508129.

# Annex A.X. WT#2 Migration and Interworking Scope Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex. WT#2: Study migration and interworking, including 1 How to support migration to 6GS. 2 How to support interworking with 5GS (including when using 5G NTN/satellite access). 3 Whether and how to support interworking with EPS (including when using 4G NTN/satellite access). NOTE 1: Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G. NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN. NOTE 3: The Work Task should consider roaming aspects.

# 1. Discussion

At SA2 #170, the work task scope of “Migration and Interworking” has been postponed due to different understandings on various aspects. This paper tries to clarify 1) the definition of the Standalone 6G architecture, 2) the scope of the combined core network nodes, 3) the scope of the WT#2. We also propose to update the work task scope and key issue accordingly.

- Definition of the stand-alone 6G architecture in SA vs RAN

One common requirement for 6G, from various sectors, is a “stand-alone 6G architecture”.  In previous discussion in SA2 and in RAN, there were lots of discussion related to stand-alone 6G architecture. Currently, there is no consensus on what the stand-alone 6G architecture means, while there is some consensus on which scenario to exclude. In fact, stand-alone 6G architecture can be interpreted in two ways:

- 6G RAN connects only to a single CN type (e.g., 6G CN).

- 6G CN supports only a single RAN type (e.g., 6G RAN).

Regarding the “stand-alone 6G architecture”, RAN 6G SID (RP-252912) clearly stated that as in below:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - … Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

RAN does not state what stand-alone architecture means or whether the 6G RAN requires separated 6G CN from 5G CN. The stand-alone 6G architecture from RAN perspective is more about excluding the EN-DC (E-UTRA-NR Dual Connectivity)-like architecture what we did from 5G (e.g., 4G RAT is used for initial access and connection management) and result in fragmented markets.

The requirement of RAN is not necessarily mean that a 6G CN should support only one RAN type. On the contrary, from SA point of view, 6G CN supporting multiple RAN types (6G-RAN and NG-RAN) does not violate the agreement (6G RAN connects to only a single CN type) in RAN.

Given that the 6G Core Network (6G CN) will inherit key 5GC features (e.g., Service-Based Interface (SBI), User Plane (UP) and Control Plane (CP) separation, Network Slicing) and will likely employ cloud-native implementation and deployment, the traditional design approach of defining a separated Core Network for each new Radio Access Network (RAN) is unlikely to be cost-effective or provide optimal operational and management efficiency for operators.

If we define a 6G CN (regardless of whether this is a new CN or enhanced 5G CN) which serves both the 6G RAN and the NG-RAN this can be an essential tool for migration from 5GS to 6GS, while utilizing full benefit of new 6G RAN/RAT. Therefore, we need to open the scope of the study of the 6G CN architecture to both:

- Common CN for NG-RAN and 6G RAN,

- New/separated/stand-alone CN for 6G RAN.

- The scope of the combined core network nodes

During previous discussion at SA2#170, there were different voices on the scope of combined CNs regarding user plane nodes (e.g., SMF, UPF) and/or control plane nodes (e.g., AMF). Some companies proposed to exclude the possibility of the combined AMF (e.g., 5G AMF, 6G AMF), as we didn’t do for the MME and the AMF. As we inherit key 5GC features and consider the cloud-native deployment, there is no sufficient justification do not define the combined AMF. Furthermore, recognizing that 6G RAT will inevitably co-exist with 5G RAT in hybrid deployments, such a combined function is vital for simplifying network operations and management.  Therefore, we propose that this work task scope include the investigation of the architecture and functionality required for the convergence of the 5G AMF and 6G AMF within the unified Core Network.

- The scope of the study

Since 6G will inevitably co-exist with 5G, Migration and Interworking constitute essential aspects of the 6G architecture definition. Accordingly, WT#2 should study not only migration and interworking scenarios but also the corresponding architectural implications, including the combined core network nodes as discussed above.

# 2. Proposal

This contribution proposes to update the TR 23.801-01 to include the scope of WT#2 and the related Key Issue. The text for WT#2 Scope on Migration and Interworking is proposed in the first change of this paper, and it is on top of the latest version of baseline S2-2508129 discussed in SA2#170. The related KI text is proposed in the second change of this paper.

**** First Change relative to S2-2508129 ****

# Annex A.X. WT#2 Migration and Interworking Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study migration and interworking, including

1	How to support migration to 6GS.

2	How to support interworking with 5GS (including when using 5G satellite access).

2.1   Study whether and how the same CN (e.g., 6G CN) supports 6G RAN and, also NG RAN.

2.2.  Study whether, and if so which, combined NFs are needed (e.g. whether to combine a 6G SMF with the 5G SMF+PGW-C, a 6G UPF with the 5G UPF+PGW-U, and a 6G AMF with the 5G AMF).

2.2   Study principles for handling services in case of mobility (e.g., inter-system, inter-RAN) between 6G RAN and NG-RAN.

2.3   Study whether and how interfaces between 6GC and 5GC are needed in different scenarios (e.g. within the same PLMN, between different PLMNs).

3	Whether and how to support interworking with EPS (including when using 4G satellite access).

NOTE 1:	Interworking with 2G/3G is not in the scope of this study. This does not preclude the case where the UE has registered to 2G/3G/4G before registering to 5GS or 6GS.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should also consider roaming aspects.

**** Second Change (all text is new) ****

# 5.X. Key Issue #X: Migration and Interworking in 6G

This key issue will study how to support migration to 6GS and interworking with 5GS (and EPS) in 6GS. The suitable architectures for each migration and interworking scenarios will be studied. For example, the set of network functions/network entities comprising a common core network for 6G RAN and NG RAN will be considered. It will be accompanied by a study on how to define the stand-alone 6G architecture, such as a common CN for 6G RAN and NG RAN or separated/stand-alone CN only for 6G RAN. Furthermore, the study on required UE capabilities and registration modes will follow.

The solutions for this key issue will include the following aspects:

- Identify the scenarios to be supported for migration to 6GS, and study how to support those scenarios.

- Study how to support interworking with 5GS (including when using 5G satellite access):

  - How the same CN type (e.g., 6G CN) supports 6G RAN and, also NG RAN.

  - Identify and address which combined NFs are needed (e.g. whether to combine a 6G SMF with the 5G SMF+PGW-C, a 6G UPF with the 5G UPF+PGW-U, and a 6G AMF with the 5G AMF).

  - Identify the principles for handling services in case of inter-system mobility between 5GS and 6GS, and study how to support session continuity during such mobility.

  - How the interfaces between 6GC and 5GC are needed in different scenarios (e.g., within the same PLMN, between different PLMNs).

- Study whether and how to support interworking with EPS (including when using 4G satellite access).

- Study whether and how to support dual registration, single registration, and/or dual-steering between 5GS and 6GS.

- Study whether and how to support multi-access sessions using 6G RAN and NG RAN.

NOTE 1:	Interworking with 2G/3G is not in the scope of this study. This does not preclude the case where the UE has registered to 2G/3G/4G before registering to 5GS or 6GS.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: This Work Task should also consider roaming aspects.