SA WG2 Meeting #171	S2-2509198

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Interim agreements for FS_Sensing_ARC KI#4

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposed the interim agreements for KI#4 of ISAC study.

# 1.  Discussions

Based on the discussion in the SA2 and the solutions documented in TR 23.700-14, this contribution proposed the interim agreements for KI#4: Sensing Data and the Associated Information Collection and Transport.

# 2.  Proposals

To apply the following change to the TR 23.700-14.

**** First Change ****

### 7.1.Y	Agreed Principles for KI#4

The following principles are agreed in this release to address the key issue on Sensing Data and the Associated Information Collection and Transport:

-	Sensing Data is provided from the Sensing Entity to the Sensing Function via the direct connection;

NOTE: Sensing Data and associated information are defined by RAN WGs.

-	Different forms of Sensing Data and associated information can be supported, based on the sensing service session configuration from the Sensing Function;

-	Sensing Entity supports one time, periodical, and event triggered Sensing Data report to Sensing Function.

**** Next Change ****

### 7.2.Z	Topics for further consideration for KI#4

The following topics requires further considerations and would not be pursued in normative phase of this release:

-	direct exposure of Sensing Data to a 3rd party AF.

**** End of Changes ****