3GPP TSG-WG SA2 Meeting #171	S2-2509219

Wuhan, China, Oct 13 - 17, 2025	(revison of S2-250)

Source:	OPPO

Title:	Proposal on Interim agreements for KI#6

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#6 for the FS_Sensing_ARC TR 23.700-14.

# 1	Discussion

Objective of KI#6 is to study how and what parameters need to be exchanged between the Sensing Entities and the Core Network

The solutions for KI#6 are documented in Solution #3, #15, #20, #27, #34, and #35.

Here is a summary of each solution in terms of how and what parameters need to be exchanged between the Sensing Entities and the Core Network:

Solution #3:

- How: The AF subscribes to a sensing service via the Core Network. The SF discovers Sensing Entities and provides them with configuration. A dedicated path is established for Sensing Entities to report 3GPP Sensing Data to the SF.

- What Parameters are Exchanged: Configuration parameters.

Solution #15:

- How: The SF maintains an inventory of Sensing Entities and their capabilities. For each service request, the SF selects and configures the appropriate Sensing Entities.

- What Parameters are Exchanged: Activate, deactivate, or reconfigure commands.

Solution #20:

- How: Focuses on provisioning general configuration parameters to Sensing Entities, separate from a specific sensing service request. This is done by a "sensing operation server" (a type of AF) via the SCF.

- What Parameters are Exchanged: The provisioned configuration parameters.

Solution #27:

- How: Provides a detailed breakdown of the possible parameters in a sensing service request and the resulting output. The SF authorizes the request, configures the Sensing Entities, collects data, and generates the result.

- What Parameters are Exchanged: Sensing operation configuration derived from the service request.

Solution #34:

- How: Introduces a Gateway Sensing Function (GSF) for discovery, authorization, and routing. The SF has a direct connection to the Sensing Entity (RAN) for configuration and data transfer.

- What Parameters are Exchanged: A two-part configuration:

- Service Parameters from AF (e.g., object type, area, SQoS).

- Parameters configured by SF (e.g., Radar Cross Section range list, object velocity range).

Solution #35:

- How: Proposes two methods for the Sensing Service Function (SSF, equivalent to SF) to send parameters to Sensing Entities (RAN).

- What Parameters are Exchanged:

- SSF -> Sensing Entities (Direct): Parameters like sensing service type, target area, duration, KPI, SSF ID.

- SSF -> AMF -> Sensing Entities (Indirect): A "Sensing Service Container" holding the parameters, along with NG-RAN ID and SSF ID.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * * Start of Changes * * * *

### 7.1.Y	Agreed Principles for KI#6

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

SF is introduced to configure gNB for the purpose of sensing operations on the gNB to fulfil service requirement and direct data connection establishment between SF and the gNB.

#### The configuration parameters include service parameters from the AF and parameters configured by the SF:

#### service parameters from the AF: sensing service type, target sensing area, sensing service QoS, sensing time parameters, and sensing contextual information

- parameters configured by the SF: radio signal processing parameters, reporting configurations, data transport information

NOTE 1: Details of data transport information are determined by conclusions of KI#4.

The parameters can be configured directly to the gNB or via AMF in a container.

NOTE 2: Configuration parameters and how they are configured to gNB by the SF will be coordinated with RAN WGS.

* * * * End of Changes * * * *