3GPP TSG-SA WG2 Meeting #171	S2-2509255

# October 13-17, 2025 Wuhan, China

Source:	Xiaomi

Title:	Interim agreement/Conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement/conclusions for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

1. Discussion

TS 23.700-14 v1.0.0 has documented 10 solutions relevant to KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection:

This key issue aims to study how to perform the Sensing Entity(ies) and Sensing Function(s) discovery and (re-)selection, in particular:

-	When required, for the discovery and (re-)selection of the Sensing Entity(ies), the study needs to address the following aspects:

-	Which entity(ies) performs the discovery and (re-)selection of the Sensing Entity(ies);

-	What the information and/or criteria are taken into account for the discovery and (re-)selection of the Sensing Entity(ies); and

-	The procedure(s) for the discovery and (re-)selection of the Sensing Entity(ies).

-	When required, for the discovery and (re-)selection of the Sensing Function(s), the study needs to address the following aspects:

-	Which entity(ies) performs the discovery and (re-)selection of Sensing Function(s);

-	What the information and/or criteria are taken into account for the discovery and (re-)selection of Sensing Function(s); and

-	The procedure(s) for the discovery and (re-)selection of the Sensing Function(s).

Based on the relevant 10 solutions, there are following observations and proposals.

Observation 1a: Sol#3, #13, #14, #15, #20, #21, #26, #28, #29, #37 propose to enhance NEF to discover and select the Sensing Function. Sol#1, #3, #6, #21, #33, #34 propose new Gateway Sensing Function to discover and select the Sensing Function.

Proposal 1: enhanced NEF is defined to support Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF) based on the parameters provided by Sensing Service Consumer (i.e., 3rd party AF) via local configuration on SF/SCF, or via querying NRF.

The parameters considered for discovery and (re-) selection of SF or SCF may include, target sensing service area (e.g., geographical location), service requirements for Sensing Service (e.g., sensing accuracy, response time, ), SF/SCF load, etc.

Observation 2: Sol#1, #3, #4, #6, #13, #15, #16, #20, #21, #22, #24,  propose SF/SCF to discover and select the SE(s) based on the parameters provided by sensing service consumer (i.e., 3rd party AF). However Sol#5, #17 proposes SF to discover and select AMF, then AMF discovers and selects the SE(s).

Proposal 2: SF or SCF is defined to discover and (re-) select of the Sensing Entity(ies) based on Sensing Service Reqeust provided by the Sensing Service Consumer (i.e., 3rd party AF) via local configuration on SF/SCF, or via querying NRF.

The parameters considered for discovery and (re-) selection of SE(s) may include, Sensing Entity(ies) Capabilities, Sensing Service Area information supported by SE.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * First Change * * * *

7	Interim Agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

To supports KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection, the following principles are concluded:

Principle 1:  enhanced NEF or new Gateway Sensing Function (GSF) is defined to support Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF) based on the parameters provided by Sensing Service Consumer (i.e., 3rd party AF) via local configuration on SF/SCF, or via querying NRF.

The parameters considered for discovery and (re-) selection of SF or SCF may include, target sensing service area (e.g., geographical location), service requirements for Sensing Service (e.g., sensing accuracy, response time, ), SF/SCF load, etc.

Principle 2a: SF or SCF is defined to discover and (re-) select of the Sensing Entity(ies) based on Sensing Service Reqeust provided by the Sensing Service Consumer (i.e., 3rd party AF) via local configuration on SF/SCF, or via querying NRF.

The parameters considered for discovery and (re-) selection of SE(s) may include, Sensing Entity(ies) Capabilities, Sensing Service Area information supported by SE.

Principle 2b: The SF or SCF is defined to discover and select the relevant AMF that is enhanced to discover and (re-) select of the SE(s) based on Sensing Service Request provided by the Sensing Service Consumer (i.e., 3rd party AF) via local configuration on SF/SCF or via querying NRF.

Principle 3a: NRF is enhanced to support the registration of SE, SF, AMF on Sensing related capabilities or information.

Principle 3b: enhanced AMF, or SF/SCF or enhanced NRF is used to support for registration of SE(s)

Editor’s Note X1: whether SF or SCF is supported will align with final conclusion of Key Issue#1.

* * * End of Changes * * * *