3GPP TSG-SA WG2 Meeting #171	S2-2509262

Wuhan, China, October 13 – 17, 2025

Source:	Samsung

Title:	Conclusion for KI#1

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim conclusion for KI#1.

1.	Discussion

The TR 23.700-67 includes total 8 solutions for KI#1 Network energy related information exposure, as excerpted in the following table:

| Sol.# | Title | Main Proposal |
| --- | --- | --- |
| 1 | Enhancement of Exposure API to include UE categorization data | Exposure of UE categories as per energy consumption in the network for UE. |
| 2 | Policy Control Considering Slice or 5GC Energy Consumption | PCF adjusts PDU session(s) of UE based on energy consumption information received from OAM. |
| 8 | Estimation of Renewable Energy consumption | Calculation of renewable energy consumption |
| 9 | Consumption energy source exposure | Expose share of the renewable energy as part of the full consumption energy |
| 10 | Energy consumption calculation enhancements | Calculation of energy consumption using a linear propoption formular. |
| 11 | Energy Consumption estimation considering idle energy consumption | Consider idel energy consumption when calculating energy consumption |
| 12 | Enhancement of energy related information exposure | Enhance exposure information to include major contributors (application ‘x’) to energy consumption for a UE. |
| 22 | Support of UP path adjustments to maximize the usage of renewable energy | SMF to reselect UPF based on renewable energy usage information from the EIF. |

2. Proposal

It is proposed to adopt the following changes to TR 23.700-67.

* * * Start of Change  (all new text)* * * *

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

The followings are agreed for KI#1 as basis for normative work:

- It is supported to expose renewable energy consumption or usage information to the consumer as requrested.

NOTE 1: The support of exposure of renewable energy information needs to be confirmed with SA5.

- The energy consumption information from the EIF or OAM can consider idle energy consumption at network nodes.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

The following bullets will be concluded based on further discussion:

- Whether information on major contributors to energy consumption (e.g., UE IDs, application information, etc.) can be included in the exposure

* * * End of Change * * * *