SA WG2 Meeting #172	S2-2509942

17 - 21 November, 2025, Dallas, USA

# Title:	[DRAFT] Reply LS on the paging in Store and Forward

# Response to:	R2-2507930/ S2-2509879, LS to SA2 on the paging in Store and Forward;

# Release:	Rel-19

# Work Item:	5GSAT_Ph3-ARC, IoT_NTN_Ph3-Core

# Source:	NEC (To be SA2)

# To:	RAN2, CT1, SA3

# Cc:	RAN3

# Contact Person:	Chunhui Zhu

# zhu_chunhui@nec.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1	Overall description

SA2 thanks RAN2 for the LS on the paging in Store and Forward. SA2 discussed the LS and would provide the following feedback.

RAN2 question:

RAN2 would like to ask SA2 if any CN initiated paging is expected from MME to RAN during the time when the feeder link is not available.

SA2 answer:

From SA2 perspective, in the RAN2 LS case, there will be no MT downlink data triggered paging, but there may be NAS protocol and/or security signalling triggered paging, e.g. MME initiates a re-authentication to create a new EPS security context by sending a SECURITY MODE COMMAND message to UE. SA2 kindly asks CT1 and SA3 for confirmation and further feedback.

SA2 would like to further clarify that according to the following specification in SA2 TS 23.401 v19.4.0, the S&F Wait Timer can be used for UE power consumption optimization including whether to listen to paging or deactivate its Access Stratum functions.

-	The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure.

…

NOTE 5:	When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions.

SA2 kindly asks RAN2 whether this S&F Wait Timer can address the RAN2 issue where the RAN2 solution is to send an indication in SIB to inform UEs that no paging (including CN initiated paging and RAN initiated paging) is expected in Store and Forward mode.

2	Actions

To RAN2, CT1, SA3:

ACTION: SA2 kindly asks RAN2, CT1 and SA3 to take the above information into account.

3	Dates of next SA WG2 meetings

SA2#173	9 – 13 Feb., 2026	     Goa, INDIA

SA2#174	13 – 17 April, 2026      Malta, MT