3GPP TSG-SA WG2 Meeting #172	S2-2510165

Dallas, US, November 17 - 21, 2025	(revision of S2-250xxxx)

Source:	Sony

Title:	[KI#2] Sol. Update, Solution 13

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution removes the Editor’s note and clarifies that the PLMN ID or NID is included when the AIOTF triggers the registration without filtering information the AF.

1. Introduction

There is an Editor’s note in the solution about providing PLMN or NID information in the paging message when an individual AIoT device is paged to trigger the device to perform initial registration.

Editor's note:	It is FFS whether and how to provide PLMN ID or NID when single AIoT Device is paged with a Temporary ID.

Solution 13 provides two ways to trigger a AIoT device to start initial registration in a network.

- The AIOTF receives a service request to trigger a set of devices based on Filtering info received from an AF.

- The AIOTF triggers, based on local configuration, all devices that are not yet registered in the network to register. In this case there is no AIoT Identification Information in the Inventory Request.

Based on that there is already two ways to get unregistered AIoT devices to register in the network, there is no need to add a third option to only trigger a specific individual AIoT device to perform initial registration. Therefore there would be no need to specify how PLMN or NID can be added to the AIoT Identification Information when Temporary ID is used.

Proposal: Remove the Editor’s note as this additional trigger option is not strickly needed in Rel-20.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change * * * *

## 6.13	Solution #13: Network Triggered Initial Registration

### 6.13.0	High-level solution principles

The key technical principles proposed in this solution are summarized below:

-	Network triggers the AIoT Device to perform initial Register using Inventory procedure to trigger un-registered devices to register.

NOTE:	Registration due to mobility or periodic registration is assumed to be autonomously done (using the DO-A procedure) by the AIoT Device after the AIoT Device has performed initial registration.

-	Reuse of Rel-19 Inventory and Command procedure to support registration of an AIoT Device as specified in TS 23.369 [3]. A Registration Indicator is included in the Inventory Request and paging message.

-	Reuse of Rel-19 security solution for authentication and command protection as specified in TS 33.369 [11].

### 6.13.1	Description

This solution proposes to reuse the Inventory procedure and command procedure to support Device registration to the network. This allows re-use of rel-19 security procedures i.e. authentication, privacy protection and command protection.

Key Procedures:

-	AF invokes a new network service to register AIoT Device(s). The AF may include e.g. External Target Area information to aid the network to select AIOTF and where perform Inventory to trigger initial registration for specific AIoT Devices.

-	The AIOTF can trigger the initial registration without an AF service invocation. In this case the AIOTF determines the periodicity and selection of NG-RAN(s) based on AIOTF implementation. The AIOTF will include PLMN ID or NID, but not the AIoT Identification Information in the Inventory Request message to trigger all AIoT Devices that are not registered in the network to register.

-	A Registration indication is included in the Inventory Request to restrict the responses to only from AIoT Devices that are not registered to the network.

-	The AIOTF uses the Inventory procedure to trigger the AIoT Device to check if it is registered to the network. The PLMN ID or NID must be provided in the paging message (see the procedure below for more details).

-	Un-registered AIoT Device(s) includes an indication to register in the Inventory Response message. As specified in rel-19 the Inventory Response message includes protected information that the network uses to derive the AIoT Device Permanent Identifier and to authenticate the AIoT Device.

-	The AIOTF sends a Command (based on rel-19 functionality) with a Registration Accept message. This message includes the necessary information for the device to enable DO-A transmission. The details of the parameters and information will be identified in solutions for the DO-A transmission and security aspects specified by SA3 for DO-A transmissions.

### 6.13.2	Procedure

Figure 6.13.2-1: Procedure for network triggered registration

0.	The AIoT device profile per AIoT Device is provisioned in the ADM.

1.	The AF invokes Nnef_AIoT_Registration Request in service operation request to the NEF. The Request includes same parameters as Nnef_Inventory Request as specified in TS 23.369 [3].

2.	The NEF may further authorize the AF request, selects AIOTF and invokes the Naiotf_AIoT_Registration service operation towards each of the selected AIOTF(s). The Request includes same parameters as Naiotf_Inventory Request as specified in TS 23.369 [3].

3.	The AIOTF receives the Naiotf_AIoT_Registration request and checks the parameters included in the request. The AIOTF may perform authorization of the AF as specified in clause 5.6 of TS 23.369 [3]. The AIOTF prepares the Inventory Request to be sent to the NG-RAN. The AIoT Identification Information to be included in the paging message includes the PLMN ID or NID, i.e. the Filtering Information includes the PLMN ID or NID part.

4.	AIOTF sends the AIoT Registration Service Response to the NEF containing the accept or reject result for the AIoT service operation request based on step 3.

5.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT service operation request as specified in clause 8.3 of TS 23.369 [3].

6.	Same as step 7 in the Inventory procedure specified in TS 23.369 [3] with the additional Registration Indicator included in the Inventory Request. If triggered by an AF the Inventory Request includes the AIoT Identification Information corresponding to the AF request.

This step can be performed by the AIOTF without receiving the service request from an AF, in this case the AIOTF determine the periodicity and selection of NG-RAN(s) based on AIOTF implementation. In this case the Inventory Request will include PLMN ID or NID but not the AIoT Identification Information to trigger all AIoT Devices that are not registered in the network.

7.	Same as step 9 in the Inventory procedure specified in TS 23.369 [3] with the additional Registration Indicator included in the paging message.

8.	The AIoT Device determines whether it matches the AIoT Identification Information as specified in step 9 of the Inventory procedure specified in TS 23.369 [3].

The AIoT Device checks the PLMN ID or NID and whether the AIoT Device is registered or not. In case the AIoT Device is not registered, the AIoT Device includes a Registration Request Indication in the AIOT NAS message. The AIOT NAS message also includes the parameters as specified in step 9 of the Inventory procedure specified in TS 23.369 [3].

9.	Same as step 10 in the Inventory procedure specified in TS 23.369 [3].

10.	Same as step 11 in the Inventory procedure specified in TS 23.369 [3].

11.	For each successful Registration Request received, the AIOTF prepares a NAS Registration Accept message and includes the NAS Registration Accept message in the Command Request message. The rest of this step is as specified in step 8 in Command procedure specified in TS 23.369 [3].

NOTE 1:	Detailed list of parameters to be included in the NAS Registration Accept message will be determined in coordination with SA3 to support DO-A message transmissions.

12.	Same as specified in step 9 in Command procedure specified in TS 23.369 [3].

13.	Same as specified in step 10 in Command procedure specified in TS 23.369 [3]. The Command Response includes the NAS Registration Complete message.

14.	Same as specified in step 11 in Command procedure specified in TS 23.369 [3]. The Command Response includes the NAS Registration Complete message.

15.	The AIOTF updates the AIoT Device Context and the AIoT device profile in the ADM that the AIoT device is registered with the network.

16.	The AIOTF reports the result of the Naiotf_AIoT_Registration request to the NEF by sending the Naiotf_AIoT_Registration Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), Registration status and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Registration Notify messages from multiple AIOTFs.

NOTE 2:	The above parameters are the same as included in the Naiotf_AIoT_Inventory Notify message.

NOTE 3:	In case multiple AFs are included in the AIoT Device Profile, then all AFs will be notified about the initial registration event.

17.	The NEF informs the AF of the result of the Nnef_AIoT_Registration request by sending the Nnef_AIoT_Registration Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), Registration status and optionally location of each AIoT Device), AF ID, [Last Report Indication]).

### 6.13.3	Impacts on Services, Entities and Interfaces

NEF:

-	Support new Nnef_AIoT_Registration service.

AIoT Device:

-	Support Registration procedure including:

1.	Verify whether it is registered in the specific network;

2.	Receive Registration Accept message; and

3.	Use the parameters included in the accept message and respond with a Register Complete message.

AIOTF:

-	Support Registration service procedure:

1.	Invocation of the service by the NEF/AF;

2.	Receiving in the NAS Inventory Response and indication that the AIoT Device requests to register in the network;

3.	Configure the AIoT Device with parameters included in the Registration Accept message.

ADM:

-	Maintain the AIoT Device Profiles with registration status information.

NG-RAN:

-	None.

AIoT Reader:

-	None.

* * * End of Change * * * *