SA WG2 Meeting #172	S2-2510647

17 – 21 Nov, 2025, Dallas, US

Source:	Samsung

Title:	[WT#1.2, Network Sharing] WT and KI update

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR proposes update WT and KI for WT#1.2 Network Sharing.

1. Discussion

The following EN for WT#1.2 network sharing remains:

Editor's note:	It is FFS whether other aspects of network sharing in 6G can be added in this clause.

For 6G, it is desirable to support network sharing to support operators to share 6G network resources in a more active and efficient manner, in order to enhance network resilience and to achieve greater reduction in CAPEX/OPEX.

As discussed in SA1, the following potential new requirement is captured and under discussion: “[PR 5.7.4.2-2] The 6G network shall provide a mechanism to support event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints).” In SA2, it should be investigated whether and what kind of impacts to system architecture or procedure are needed to support that requirement. At least, it is required to be discussed whether and how network sharing for disaster condition in 5G can be supported in 6G.

From network resilience point of view, as discussed in SA1 (see clause 5.4.1 of TR 22.870), global telecommunications sector has experienced numerous/significant incidents which led to internet outage and disruptions in industry applications. 6G network should be self-adaptive and self-healing with enabling zero-touch network construction, e.g. in the case of network failure due to overcrowding or disaster situations.

2. Proposals

Considering the above discussion, this pCR contributes to resolve the EN and accordingly update the KI description with proposing the following changes in TR 23.801-01.

**** First Change ****

A.1.2.1	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

-	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

-	Investigate whether and how to support MOCN and/or INS in 6G for the scenario of event-triggered network sharing, e.g., disaster occurrence, network failure, overloaded situation, resource constraints.

NOTE:	Whether and how to investigate system impacts for the scenario of event triggered network sharing will be discussed in coordination with SA1.

**** Next Change ****

5.1	Key Issue #1: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

1)	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

2)	Investigate whether and how to support MOCN and/or INS in 6G for the scenario of event-triggered network sharing, e.g., disaster occurrence, network failure, overloaded situation, resource constraints.

NOTE:	Whether and how to investigate system impacts for the scenario of event triggered network sharing will be discussed in coordination with SA1.

**** End of Changes ****