3GPP SA WG2 Meeting #170	S2-2507459_was_6429

Stor-Göteborg, Sweden, 25 - 29 August 2025

Title:	[DRAFT] Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	Reply LS on signalling feasibility of dataset and parameter sharing (R2-2506108/ S2-2504512)

Release:	Release 19

Work Item:	NR_AIML_air-Core FS_NR_AIML_air_Ph2

Source:	Samsung [to be SA2]

To:	RAN2

Cc:	RAN, RAN1, RAN3, SA, SA2, SA3, SA5

Contact Person:

Name:	Hyesung Kim

E-mail Address:	hs1207.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks the LS on signaling feasibility of dataset and parameter sharing and provides feedback on the following RAN2’s assumption on non-OTA solutions:

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1).  From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption.

SA2 Feedback:

SA2 confirms that the above assumption is not correct. SA2 agrees that non-OTA solutions cannot be supported within Rel-19 existing architecture framework. It requires enhancements on specific capabilities, including network exposure capabilities and how to exchange of AI/ML-related data between CN and gNB. SA2 also would like to note the followings:

- RAN3 confirmed that the exchange of AI/ML-related data between the CN and gNB is not supported in R19 (as described in R3-253961);

- Rel-19 is frozen now for SA2.

To study non-OTA candidate solutions addition to the existing architecture, SA2 requires RAN2 / RAN Plenary to provide confirmation of the requirement. The decision is also subject to SA/SA2 planning.

- SA2 would like to ask RAN2 about the difference between option 3 and the other two options. In particular, how option 3 (gNB -> OAM/CN -> UE-side training entity) and option 2 (CN -> UE-side training entity) are different from the CN entity perspective?

2. Actions:

RAN2:

ACTION: 	SA2 kindly asks RAN2 to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October, 2025	Wuhan, CN

TSG-SA2 Meeting #172		17-21 November, 2025	Dallas, US