# 3GPP TSG-SA3 Meeting #125																			draft S3-254279-r2

# Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon, Samsung

Title:	Add evaluation in Sol#6 of TR 33.700-30

Document for:	Approval

Agenda item:	5.2.9

Spec:	TR 33.700-30

Version:	0.1.0

Work Item:	FS_5GSAT_Ph4_SEC

Comments

This proposal is to add evaluation in Sol#6 of TR 33.700-30.

* * * First Change * * * *

### 6.6.3	Evaluation

This solution addresses the Key Issue #1, and it applies for S&F operations with multiple satellites. In this solution,

the UE can exchange data with multiple satellites efficiently without security risk.

The solution has the following impacts:

This solution requires the MME-ground and the UE to derive new keys (i.e. KASME* derivation based on KASME) for different satellites and the UE needs to maintain multiple NAS COUNTs. This solution requires the key transfer from MME-ground to MME-onboard.

If the UE interacts with a new satellite, it computes a new KASME* and derives new NAS keys based on the same NAS security algorithms.

The same UE context cannot be re-used across satellites because new NAS keys need to be generated by the UE.

Each MME on-board needs to maintain a separate pair of NAS COUNTs.

Editor’s Note: The detail on securing NAS messages using different NAS keys during handover-like process is FFS.

* * * End of Changes * * * *