# 3GPP TSG-SA3 Meeting #125																			Draft S3-254283-r2

# Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon, Samsung

Title:	Address ENs in Sol#6 of TR 33.700-30

Document for:	Approval

Agenda item:	5.2.9

Spec:	TR 33.700-30

Version:	0.1.0

Work Item:	FS_5GSAT_Ph4_SEC

Comments

This proposal is to address ENs in Sol#6.

* * * First Change * * * *

### 6.6.2	Solution details

Based on the existing authentication procedures, this solution proposes to use different NAS keys when UE exchanges data with multiple satellites.

Figure 6.6.2-1 Enhanced NAS security for multiple satellites in S&F mode

SAT#1 has available Service Link.

- The UE sends the Attach Request to SAT#1.

- If SAT#1 does not have context to authenticate the UE, then sends the Attach Reject.

SAT#1 has available Feeder Link.

- SAT#1 sends the Attach Request to the MME-ground.

- The MME-ground obtains authentication data including KASME, as defined in TS 33.401 [3].

- The MME-ground determines to use SAT#1 to serve UE, then the MME-ground calculates KASME1* by using KASME and SAT Id of SAT#1.

- The MME-ground distributes KASME1* for SAT#1 during the transmission of AV.

SAT#1 has available Service Link.

- The authentication procedure is completed, as defined in TS 33.401 [3].

- SAT#1 derives NAS keys based on the KASME1* using existing mechanism as defined in TS 33.401[3] and sends the NAS security mode command integrity protected.

- The UE calculates KASME1* using the same method as the MME-ground in step5, and further derives the NAS keys using existing mechanism as defined in TS 33.401[3], then the UE verifies the NAS security mode command.

- If successfully verified, the UE sends the NAS security mode complete to SAT#1.

- After the NAS SMC procedure, the UE and SAT#1 send protected NAS messages.

SAT#2 has available Feeder Link.

- The MME-ground determines to use SAT#2 to serve the UE, the MME-ground calculates KASME2* by using KASME and SAT Id of SAT#2.

- The MME-ground distributes KASME2* for SAT#2.  Then SAT#2 derives the NAS keys by using KASME2*.

SAT#2 has available Service Link.

- The UE establishes RRC connection with SAT#2 and calculates KASME2* using the method as the MME-ground in step12, and further derives the NAS keys by using KASME2*.

NOTE 1:  The UE obtains the SAT Id of SAT#2 broadcast by the satellite SAT#2 to derive KASME2*.

- The UE sends a protected initial NAS message to SAT#2. The successful processing of the initial NAS message by the satellite SAT#2 activates the NAS key between the UE and the satellite SAT#2. The UE and SAT#2 exchange protected NAS messages.

NOTE 2: As described in TS 23.401[2], the MME-ground together with the associated MME-onboard(s) behave jointly as a single MME entity. For multiple satellites, assume MME-onboards have the same list of ordered NAS security algorithms. After NAS SMC, the selected NAS security algorithms could be synchronized for MME-onboards. Each satellite/UE pair maintains independent COUNTs.

NOTE 3: As Store-and-Forward satellite operations involve sparse, infrequent transmissions, spread across multiple satellites, COUNT wrap-around can only occur over a substantial amount of time. It is left to network policy to determine when the MME ground re-authenticates the UE to avoid COUNT wrap around.

NOTE 4: To secure NAS messages using different NAS keys during handover-like processes in Store-and-Forward satellite operations, the UE handles satellite changes as cell reselection events and performs a protected tracking area update procedure using NAS keys.

Editor’s note: The establishment of AS security in this solution is FFS.

* * * End of Changes * * * *