3GPP TSG-SA3 Meeting #125	S3-254485-r1

# Dallas, US, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2203 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GC Signaling Traffic Monitoring Security Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MonStra-Sec |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 33.501 section 6.2 describes several cryptographic keys. Mask these keys before transporting copies of signaling traffic to Signaling Traffic Monitoring systems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When security key (s) is transported in signaling between 5GC NF, the key (s) value is to be removed, masked and preserve the length of the field from the copies of the signaling traffic sent to Signaling Traffic Monitoring systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security risk of exposing cryptographic keys outside the 5GC environment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Change ***

### 5.9.4	Requirements for monitoring 5GC signaling traffic

#### 5.9.4.1	Security requirements for the configuration of signalling monitoring

The 5G shall support mutual authentication between the STM Management Producer at the 5GC (for configuration/activation of the functionality) and the STM Management Consumer.

The 5G shall support authorization to the STM Management Consumer.

NOTE 1: Local policy-based authorization should be supported by the STM Management Producer, according to TS 28.533 [54].

The 5G shall support integrity protection, replay protection and confidentiality protection for communication between the STM Management Producer at the 5GC and the STM Management Consumer.

NOTE 2: If interface between the STM Management Producer and the STM Management Consumer is trusted (e.g. physically protected), it is for the PLMN-operator to decide whether to apply integrity protection, replay protection and confidentiality protection for communication.

#### 5.9.4.2	Security requirements for the streaming of signalling monitoring data

The 5G shall support mutual authentication between the STM Data Producer at the 5GC and the STM Data Consumer at the Monitoring system.

NOTE: The Monitoring system is outside the scope of the present document.

The 5G shall provide confidentiality protection, integrity protection and replay protection of the interface between the STM Data Producer at the 5GC and the STM Data Consumer at the Monitoring system.

Keys stored inside a 5GC NF shall never leave a secure environment within the 5GC NF for Signalling Traffic Monitoring purposes. As the STM Data Consumer may reside outside the operator’s secure 5GC domain, when security key(s) transported on SBA control plane signalling messages are included in the copies of SBA control plane signalling traffic to the STM Data Consumer at the Monitoring system, the key value(s) shall be masked with a string by preserving the length of the key value (s).

*** End of Changes ***