# 3GPP TSG-SA3 Meeting #125														S3-254707

# Dallas, US, 17 – 21 November 2025																[revision of S3-254073]

Source:	Nokia

Title:	Pseudo-CR on Solution for Detection of compromised MWAB nodes

Document for:	Approval

Agenda item:	5.2.14

Spec:	3GPP TR 33.724

Version:	V0.1.0

Work Item:	FS_5G_WAB_SEC

Comments

This contribution proposes a solution to detect compromised MWAB nodes to address Key Issue #2.

* * * First Change * * * *

## 6.Y	Solution #Y: Solution for Detection of compromised MWAB nodes

### 6.Y.1	Introduction

As mentioned in Key Issue #2 “Security Protection of Compromised WAB Nodes and Core Network Measures”, “core network components may not be equipped to detect anomalous behavior from compromised WAB-gNBs, due to the decentralized and mobile nature of such nodes. The compromised WAB nodes could lead to topology poisoning, signalling storms, or user-plane hijacking.”

This solution proposes to configure a legitimate MWAB node with a pre-configured code/certificate in which UEs can verify the legitimacy of a MWAB node upon connection.

### 6.Y.2	Solution details

The following steps are followed:

- The MWAB node manufacturer configures the MWAB node with a certificate verifying its identity and enrolls with the Operator’s network.

- Operators maintain a map of MWAB nodes with their expected location, and associated code for each MWAB node.

- AMF sends the UE an authentication request with additional information such as location, signal strength, and the public key of the MWAB node.

- The 5GC sends the certificate received from the MWAB node to the UE and the UE sends a verification request to the MWAB node.

- The MWAB node responds to the UE with the decoded message signed using a certificate verifying its identity and the UE requests the 5GC to verify the MWAB node after the NAS security context has been established.

- If verification is successful, the 5GC informs the UE.

- Else, if verification is unsuccessful, then the 5GC informs the UE, and the UE disconnects and informs the operator’s 5GC.

UE

MWAB

5

GC

1

. Encoded

Random

Msg (Secure NAS)

. Request to decode

2

Encoded

Random

Msg

3

. Decoded

Random

Msg

4

. Decoded Random Msg (Secure NAS

)

OAM / Operator

5

. Verify

Random Msg

6

. OK/NOK MWAB device verification

with UE location (Secure NAS)

UE

0

. Pre-configured

Code / Certificate

)

(

To 5GC + MWAB

UE

MWAB

5

GC

1

. Encoded

Random

Msg (Secure NAS)

. Request to decode

2

Encoded

Random

Msg

3

. Decoded

Random

Msg

4

. Decoded Random Msg (Secure NAS

)

OAM / Operator

5

. Verify

Random Msg

6

. OK/NOK MWAB device verification

with UE location (Secure NAS)

UE

0

. Pre-configured

Code / Certificate

)

(

To 5GC + MWAB

The figure below shows a high-level procedure for this solution.

An example of the code used to identify the MWAB node is a combination of the hardware serial number and software version number, which is unique for each MWAB node and can be verified by the operator’s 5GC according to the expected MWAB node in that location.

Editor’s Note: How does the solution prevent an attacker from replaying or spoofing MWAB certificate, particularly before a secure NAS context has been established is FFS.

Editor’s Note: Whether and how to ensure the authenticity of the MWAB node’s location is FFS.

### 6.Y.3	Evaluation

TBD

* * * End of Changes * * * *