# 3GPP TSG-SA3 Meeting #125																			S3-254072

# Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Evaluation for Solution for NAS COUNT synchronization in store-and-forward operations

Document for:	Approval

Agenda item:	5.2.9

Spec:	3GPP TR 33.700-30

Version:	V0.1.0

Work Item:	FS_5GSAT_Ph4_SEC

Comments

Adding evaluation clause for the Solution for NAS COUNT synchronization in store-and-forward operations

* * * First Change * * * *

### 6.7.3	Evaluation

This solution addresses Key Issue #1.

It has the following advantages:

- This solution proposes the use of a new “Satellite access information” which can be included in the initial UE message sent from satellite eNB to MME.

- Also, this solution recommends that the MME on-ground maintains the latest UL and DL NAS COUNTS considering that MME on-ground is the common entity between MME on-boards of multiple satellites. This can be a simple to implement solution. Note that the security between MME on-board and MME on-ground is not in scope of 3GPP and is assumed to be implemented by the operators.

- The coordination between MME-on-board and MME-on-ground regarding NAS count synchronization is out of scope of 3GPP.

- From the UE’s perspective, MME-on-board and MME-on-ground are a single entity, and this is consistent with the architecture specified for split MME.

- NOTE: This solution has some additional signalling between MME on-board and MME on-ground for synchronizing the DL and UL NAS COUNTs.

- NOTE: The UE is agnostic to the satellite actions and decisions and is unaware of the split MME architecture and the roles of the individual MME components.

This solution impacts the MME on-board and MME on-ground and the signalling between these entities.

* * * End of Changes * * * *