3GPP TSG-SA3 Meeting #125	S3-254679

Dallas, US, 17 – 21 November 2025

Source:	Vodafone

Title:	pCR to TS 33.502 – Detecting SBA layer parameters change

Document for:	Approval

Agenda item:	5.1.1

Spec:	3GPP TS 33.502

Version:	0.2.0

Work Item:	SECHAND

Comments

Most, if not all, the security related events that can occur at the SBA layer can have their origin in an erroneous or malicious configuration related (but not limited) to:

- Changes of the communication models (e.g. an NF expected to use indirect mode with delegated discovery is changed to use direct mode).

- Changes to local configurations as alternative to SBA dynamic procedures (e.g. static IP addresses, discovery, etc)

- Changes related to the types of NFs allowed to access a given NF instance (e.g. a UDR instance is allowed to be accessed by non-intended NFs)

- Changes related to authorizing scopes assigned to an NF-Consumer (e.g. an NF-Consumer of certain type is allowed to access another NF resources without restriction).

- Changes related to parameters used to establish security link between SEPPs

- Changes related to the NF status (e.g. an NF “canary release” condition is removed)

- Changes to security trust condition of AFs (e.g. an AF is changed from untrusted to trusted condition)

These example changes can be executed in conjunction or independently from other changes, leading to abnormal situations in the SBA layer with a potential malicious origin.

This pCR proposes adding a security related event every time an NF SBA related parameters are changed/updated. In addition, the time of the last NF configuration should be appended to the common information elements included in clause 6.2 for the events specified in clause 6.

* * * First Change * * * *

## 5.x		Other requirements

The security related events collection entity shall have access to configuration related information of the NF.

Editor’s Note: The above requirement needs to be refined.

NOTE x: 	Configuration related information of the NF can be available through the O&M CM (Configuration Management) northbound interfaces.

* * * Next Change * * * *

# 6	Security related Events

Editor’s Note: This clause addresses the list and description of the events as well as naming convention for the events.

## 6.X	Security events related to SBA parameters configuration

An NF configuration is received in which related SBA level parameters are changed/updated.

Editor’s Note: The details of this event is for FFS.

* * * End of Changes * * * *