3GPP TSG-SA3 Meeting #123	draft_S3-252996-r4

- Goteborg, Sweden, 25 - 29 August 2025           merger of S3-252557, S3-252657, S3-252729, S3-252792, S3-252810, S3-252834, S3-252835, S3-252840, S3-252855, S3-252857, S3-252864, S3-252878, S3-252886

Source:	Huawei, HiSilicon, Lenovo

Title:	Update to requirement on device

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

In SA#108, Way Forward on AIoT device credentials storage was endorsed as below in SP-250851.

“for Rel-19, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network) e.g. SNPN, and the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501). This bullet means that no interconnection exists between AIoT systems and PLMNs ;”

This contribution proposes to address the editor’s note in draft TS 33.369 accordingly.

**** START OF 1st CHANGE *****

# 4	Security requirements for AIoT service

***** END OF 1st  CHANGE *****

**** START OF 2nd CHANGE *****

## 4.2	Security Requirements

### 4.2.1	Requirements on the device Type 1 in Topology 1

***** END OF 2nd  CHANGE *****

**** START OF 3rd CHANGE *****

#### 4.2.1.2 	Requirements related to authentication between device and network

The AIoT device shall support

a method for pseudo-random bit generation.

Editor’s Note: Further cryptographic primitives are FFS.

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection and replay protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

4.2.1.4	Requirements for identifier privacy

The device shall/may support a mechanism for the use of temporary IDs.

Editor’s Note: this requirement will be revisited.

***** END OF 3rd CHANGE *****