3GPP TSG-SA3 Meeting #123	draft_S3-252997-r3 was merger of S3-252608,

S3-252592,S3-252658,S3-252854

# Goteborg, Sweden, 25 – 29 August 2025

Source:	ZTE

Title:	Update the clause 4.2.2 Requirements on the AIOTF

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

The requirements on authentication for the AIOTF needs to be added.

Proposed Changes

* * * First Change * * * *

4.2.2	Requirements on the AIOTF

.

The AIOTF shall authenticate the AIoT device.

The AIOTF shall support confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following ciphering algorithms:

NEA0, 128-NEA2 as defined in Annex D of the TS 33.501 [5].

Confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF is optional to use.

The AIOTF shall support integrity protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following integrity algorithms:

128-NIA2 as defined in Annex D of the TS 33.501 [5].

Integrity protection of AIoT NAS Command request and response between the AIoT device and the AIoTF is mandatory to use.

The AIOTF shall support selection of confidentiality and integrity algorithms for protecting AIoT NAS Command request and response between the AIoT device and the AIOTF based on operator’s local policy.

The AIOTF shall support a mechanism for the use of temporary IDs.

The  AIOTF shall support resynchronization of desynchronized temporary IDs.

* * * End of Changes * * * *