- 3GPP TSG-SA3 Meeting #124	draft_S3-253264-r2

- Wuhan, China, October 13 - 17, 2025	revision of S3-25yyyy

- Source:	vivo

- Title:	New Key Issue on Key Hierarchy

Document for:	Approval

Agenda Item:	5.3.2

# 1	Decision/action requested

Approve the pCR on new key issue for TR 33.771.

# 2	References

# 3		Rationale

This contribution proposes a new key issue on key hierarchy.

# 4	Detailed proposals

Start of 1st Change

## 5.X	Key issue #X: AEAD Keys

### 5.X.1	Key issue details

The AEAD algorithms differ from the current set of algorithms as they use a single key for both encryption and integrity protection. However, the existing key hierarchy does not include single keys to be used by AEAD algorithms.

As described in clause 6.2 of TS 33.501 [xx], the existing 5G key hierarchy is derived from a long-term key to NAS, RRC, and UP keys. These keys follow a structure where two separate 128-bit keys are used: one for encryption and the other for integrity protection.

In contrast, AEAD algorithms utilize a single key to perform both encryption and integrity protection.

The key issue is to study how the key hierarchy needs to be enhanced to cover the key need for the AEAD algorithms.

### 5.X.2	Security threats

Inappropriate key hierarchy can lead to breach of confidentiality or integrity.

### 5.X.3	Potential security requirements

The key hierarchy enhancement may need enhancement to include keys for the AEAD algorithms.

NOTE: The key generation procedure is out of scope of the present document.

End of 1st Change