- 3GPP TSG-SA3 Meeting #124	draft_S3-253266-r1

- Wuhan, China, October 13 - 17, 2025	revision of S3-25yyyy

- Source:	vivo

- Title:	New Key Issue on Operational Sequence of AEAD

Document for:	Approval

Agenda Item:	5.3.2

# 1	Decision/action requested

Approve the pCR on new key issue for TR 33.771.

# 2	References

# 3		Rationale

This contribution proposes a new key issue on operational sequence of AEAD.

# 4	Detailed proposals

Start of 1st Change

## 5.X	Key issue #X: Operational Sequence of AEAD

### 5.X.1	Key issue details

The operational sequence for the AEAD algorithm follows a specific pattern: first, encryption is applied to the plaintext, transforming it into ciphertext. Subsequently, integrity protection is performed by generating a MAC from the already encrypted ciphertext, a method known as Encrypt-then-MAC (EtM).

In the realm of NAS protection, this sequence aligns with the process utilized by AEAD algorithms, adhering to the Encrypt-then-MAC approach to maintain the integrity of the data after encryption. Conversely, within PDCP protection, the process diverges from AEAD operation. Instead, PDCP employs a MAC-then-Encrypt (MtE) strategy, whereby integrity protection is performed first by generating a MAC from the plaintext, followed by the encryption of both the MAC and the plaintext (see figure below).

Figure 5.X.1-1. Legacy AS security protection with both ciphering and integrity protection

The differences in these operational sequences highlight the need for a careful evaluation when implementing these algorithms for AS Security.

### 5.X.2	Security threats

N/A.

### 5.X.3	Potential security requirements

N/A.

NOTE:	Coordination with RAN is needed.

End of 1st Change