3GPP TSG-SA3 Meeting #124	draft_S3-253376-r1

- Wuhan, China, 13 – 17 October 2025                                  merger of S3-253376 and S3-253303

Source:	Huawei, HiSilicon, OPPO

Title:	Protection of information to support DO-A Capable AIoT Devices during AIoT service communication

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.0.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

It is proposed to approve the new key issue on information protection of DO-A procedure.

**** START OF 1st CHANGE *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[xx]	3GPP TS 22.369: "Service Requirements for ambient power-enabled IoT".

[yy]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

***** END OF 1st CHANGE *****

**** START OF 2nd CHANGE *****

## X.Y	Key Issue #Y: Protection of information to support DO-A Capable AIoT Devices during AIoT service communication

### X.Y.1	Key issue details

As per TS 22.369 [xx], Ambient power-enabled IoT (AIoT) services aim to support various use cases, including inventory taking, sensor data collection, asset tracking, and actuator control. These services intended to operate with lower power consumption and complexity than the existing IoT technologies such as eMTC, NB-IoT, and RedCap. To fulfil these requirements, AIoT devices require a communication capability.

From a security perspective, security mechanisms to protect the information transmitted during AIoT service communication need to be supported. Failure to provide such security mechanisms will lead to various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information during AIoT service communication.

### X.Y.2	Security threats

In addition to the command operation (e.g., write, read) as specified in TS 23.369 [yy], DO-A Capable AIoT Device can send data to the AIOTF autonomously. The following threats are still applicable:

An attacker may acquire data transmitted to/from AIoT devices by eavesdropping messages if the communication of AIoT service is not confidentiality protected.

An attacker may manipulate information during communication of AIoT service if the communication of AIoT service is not integrity protected.

An attacker may replay a message if replay protection is not activated.

### X.Y.3	Potential security requirements

The 5G system shall provide the means to ensure confidentiality, integrity and/or anti-replay of information for AIoT services.

***** END OF 2nd CHANGE *****