- 3GPP TSG-SA3 Meeting #124	draft_S3-253706-r1

- Wuhan, China, October 13 - 17, 2025	revision of S3-253258

- Source:	vivo

- Title:	New Solution for Security of UE connection setup with Data Collection NF

Document for:	Approval

Agenda Item:	5.2.6

# 1	Decision/action requested

Approve the pCR on new solution for AIML_CN_Ph2_SEC.

# 2	References

# 3		Rationale

This contribution proposes a new solution for security of UE connection setup with Data Collection NF

# 4	Detailed proposals

Start of 1st Change

## 6.Y	Solution #Y: Security of UE connection setup with Data Collection NF

### 6.Y.1	Introduction

This solution addresses requirements of key issue #1.

For authorization and user consent check between UE and data collection NF, it proposes that the entity who selects UE for data collection is deemed as enforcement point. Especially for user consent check, the existing mechanism can be reused.

For authentication and communication protection, it proposes that 3GPP network sends security parameters (e.g. PSK) to the UE in protected RRC/NAS message and the UE uses the security parameters to establish secure connection (e.g. TLS) with the DCF for UP data transferring.

### 6.Y.2	Solution details

Figure 6.Y.2-1 Security of UE connection setup with Data Collection NF

- Data consumer (e.g. UE model training entity server) requests UE data collection to DCF.

- DCF retrieves UE subscription data from UDM. The subscription data includes:

  - User consent data: existing user consent parameters can be reused.

  - Authorization profile: whether UE is allowed for exposing specific data to specific data consumer.

The DCF can be enforcement point for authorization and user consent check if it decides that DCF is used for UE selection for data collection.

- The DCF sends security parameters (e.g. PSK) to the RAN/AMF. The DCF may also send UE subscription data to the RAN/AMF to enforce the authorization and user consent check if it decides that RAN/AMF is used for UE selection for data collection.

- The RAN/AMF sends security parameters to the UE. The security parameters are protected by RRC/NAS mechanism.

- The UE establishes a PDU session as depicted in clause 7.1.1 of TR 23.700-04 [2].

- The UE establishes a secure connection using the security parameters to the DCF, e.g. the UE uses PSK to establish a secure TLS connection with the DCF.

- The UE reports UP data in the secure connection to the DCF.

- The DCF reports UP data to the Data consumer.

### 6.Y.3	Evaluation

TBA

Editor’s Note:	The evaluation is made based on SA2 conclusion.

End of 1st Change