3GPP TSG-SA3 Meeting #125	S3-254050-r3

# Dallas, USA, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2199 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PRINS Refinement on Trust Anchoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | RefinePRINS |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GSMA NRG sent an LS (S3-252539) to 3GPP SA3 requesting further refinement of PRINS to make it easier for roaming intermediaries (RIs) to support PRINS and its end-to-end application layer security property. Those changes are to implement the refinements of PRINS requested by GSMA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the trust anchoring mechanism in order to enable flexibility of assigning roaming relation routing to different RIs while still limiting which roaming relations can be served via which RIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRINS adoption is may be perceived as too inflexible by both MNOs and RIs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of 1st Change * * * *

### 13.1.2	Protection between SEPPs

TLS shall be used for N32-c connections between the SEPPs.

The SEPP shall maintain a set of trust anchors. For N32-c and N32-f when the selected security method is "TLS", each trust anchor consists of a list of trusted root certificates and a list of corresponding PLMN-IDs. For NDS/IP or TLS VPN used for N32-f when the selected security method is "PRINS", each trust anchor consists of a list of trusted root certificates and a list of unique identifiers for a given Roaming Intermediary (RI). Any given PLMN-ID and any given RI identifier shall appear in at most one trust anchor.

The SEPP should maintain an ‘RI-to-PLMN mapping’, i.e. a local configuration for each RI that lists the PLMN-IDs of the PLMNs for which roaming via that particular RI has been enabled contractually.

NOTE 3: Void.

During N32-c connection setup, the SEPP shall map the PLMN-IDs as extracted from the remote SEPP end entity certificate to the associated trust anchor for the purposes of certificate chain verification. Only the root certificates in the associated list shall be treated as trusted during certificate chain verification. If the remote SEPP end entity certificate contains multiple PLMN-IDs that are mapped to different trust anchors, then that certificate shall be rejected.

Operator Group Roaming Hubs SEPPs are equivalent to a network operator SEPP when they are in the same security domain and are not considered Roaming Intermediaries as detailed in this clause. The communication between a group network operator's SBA network border element and the Operator Group Roaming Hub SEPP is out of scope of the present document.

If there are no Roaming Intermediaries between the SEPPs, TLS shall be used for N32-f connections between the SEPPs. Different TLS connections are used for N32-c and N32-f. If there are Roaming Intermediaries which only offer IP routing service between SEPPs, either TLS or PRINS (application layer security) shall be used for protection of N32-f connections between the SEPPs. PRINS is specified in clause 5.9.3 (requirements) and clause 13.2 (procedures).

If TLS is selected, the SEPP shall correlate the N32-f TLS connection with the N32-c connection. If the peer network is a PLMN, the SEPP compares the PLMN-IDs contained in the SEPP TLS certificates used to establish the N32-c and N32-f connections. Specifically, if the certificate used for N32-f contains one or more PLMN-IDs that are not contained in the TLS certificate used for the corresponding N32-c, the N32-f certificate shall be rejected. If the peer network is an SNPN, the SEPP compares the SNPN-ID contained in the SEPP TLS certificates used to establish the N32-c and N32-f connections.

The SEPP shall check whether the PLMN-IDs in the header and JSON fields, if any, of incoming N32-f messages are abnormal by matching the PLMN-ID(s) in the relevant trust anchor, remote PLMN-ID(s) in the N32-f context, or in the relevant RI-to-PLMN mapping. More precisely, if TLS is used for N32-f, then the relevant trust anchor is the one selected during the setup of the correlated N32-c connection and, if PRINS via an RI is used, then the matching is performed against the relevant RI-to-PLMN mapping where the RI is the remote endpoint of the NDS/IP, TLS VPN or HTTPS connection. The SEPP should support a mode of logging where mismatches are logged.

If there are Roaming Intermediaries which, in addition to IP routing, offer other services that require modification or observation of the information and/or additions to the information sent between the SEPPs, PRINS shall be used for protection of N32-f connections between the SEPPs.

NOTE 1a:	The procedure specified in clause 13.5 for security mechanism selection between SEPPs allows SEPPs to negotiate which security mechanism to use for protecting NF service-related signalling over N32, and provides robustness and future-proofness, e.g. in case new algorithms are introduced in the future.

If PRINS is the selected security method for N32-f interface, one of the following additional transport protection methods shall be applied between SEPP and Roaming Intermediary for confidentiality and integrity protection:

-	NDS/IP as specified in TS 33.210 [3] and TS 33.310 [5], or

-	TLS VPN with mutual authentication following the profile given in clause 6.2 of TS 33.210 [3] and clause clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks, with the restriction that it shall be compliant with the profile given by HTTP/2 as defined in RFC 9113 [47].

During NDS/IP or TLS VPN connection setup with an RI for the purpose of establishing N32-f or N32-c via the HTTP CONNECT method , the SEPP shall map the RI identifiers as extracted from the RI end entity certificate to the associated trust anchor for the purposes of certificate chain verification. Only the root certificates in the associated list shall be treated as trusted during certificate chain verification. If the RI end entity certificate contains multiple RI identifiers that are mapped to different trust anchors, then that certificate shall be rejected.

NOTE 1:	Void

NOTE 2:	Void.

NOTE 4: Whether or not a given RI identifier includes the PLMN-IDs that correspond to the roaming partners that are reachable via that RI, is not specified in 3GPP.

* * * End of 1st Change * * * *