3GPP TSG-SA3 Meeting #125	S3-254060-r1

# Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Solution proposal Symmetric key SUCI

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.2.0

Work Item:	FS_PQC

Comments

This pCR is introducing the Solution proposal Symmetric key SUCI procedures.

The proposed solution is to be added into the TR study on supporting the PQC [1].

The revision1 includes new added Editor’s Notes.

[1]			3GPP TR 33.771, “Study on Transitioning to Post Quantum Cryptography (PQC)”

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ANSSI			Agence Nationale de la Sécurité des Systèmes d'Information

CA			Certification Authority

CBOR			Concise Binary Object Representation

COSE			CBOR Object Signing and Encryption

CRL			Certificate Revocation Lists

CRQC			Cryptographically Relevant Quantum Computer

DSA			Digital Signature Algorithm

ECC			Elliptic Curve Cryptography

ECDH			Elliptic Curve Diffie–Hellman key Exchange

ECIES			Elliptic Curve Integrated Encryption Scheme

FN-DSA			Fast-Fourier Transform over NTRU-Lattice-Based DSA

HBS			Hash-Based Signature

HQC			Hamming Quasi-Cyclic

HSS			Hierarchical Signature System

IKEv2			Internet Key Exchange Protocol Version 2

JSON			JavaScript Object Notation

JWE			JSON Web Encryption

JWS			JSON Web Signature

KEM			Key Encapsulation Mechanism

MIKEY-SAKKE			Multimedia Internet KEYing – Sakai-Kasahara Key Encryption

ML-DSA			Module-Lattice-Based DSA

ML-KEM			Module Lattice-Based Key-Encapsulation Mechanism

NCSC 			National Cyber Security Centre

NSA			National Security Agency

NSM			National Security Memorandum

NTRU			Nth-degree Truncated Polynomial Ring Units

PKI			Public Key Infrastructure

PKIX			Public Key Infrastructure X.509

PQC			Post-Quantum Cryptography

PRNG			Pseudo Random Number Generator

SA			Security Association

SDO			Standards Development Organizations

SECG			Security Engineering & Consulting Group

SLH-DSA			Stateless Hash-Based DSA

SUCI			Subscription Concealed Identifier

TLS 1.2			Transport Layer Security Version 1.2

TLS 1.3			Transport Layer Security Version 1.3

UDM			Unified Data Management

XMSS			eXtended Merkle Signature Scheme

* * * Next Change * * * *

## 7.2.1.Y	Solution Y: Symmetric crypto based SUCI

Overview: The UDM is creating a collection of relevant SUPI values. The UDM is encrypting by using symmetric crypto each of SUPI values from the list and is sending these to the UE. The UE is appending the selected encrypted SUPI and is hashing the encrypted SUPI together with the concatenated hashed Key KSUPI. The UDM can verify the authenticity of the SUPI. The UDM can verify the authenticity of the UE, and in successful case the UDM-UE auth can be processed.

### 7.2.1.Y.1	 Introduction

This solution addresses the key issue#1.

### 7.2.1.Y.2 	Solution details

Editor’s Note: The description of a resynchronisation procedure is FFS.

Editor’s Note: It is FFS about the first registration procedure.

Editor’s Note: It is FFS How are new encrypted SUPIs provisioned to UEs.

Editor’s Note: It is FFS How does the UDM regenerate the new symmetric keys for Pseudonyms.

Editor’s Note: Resynchronization of desynchronized RANDs is FFS.

Editor’s Note: Impact on fulfilling LI requirements is FFS.

Overview of Encrypted SUCI in UDM:

Figure 7.2.1.Y.2-1: Encrypted SUCI in UDM (overview)

In the above Figure 7.2.1.Y.2-1 the list of Encrypted SUCI is generated in UDM is shown. The UDM will first generate the key KSUPI using hash of SUPI, long term credentials of Subscriber K, RANDSUPI. Also, in parallel there will be list of Random numbers (RANDSUCI#1, RANDSUCI#2, etc) are generated using PRNG (Pseudo Random Number Generator). Use the newly generated key KSUPI to encrypt all the RANDSUCI#1, RANDSUCI#2, etc, to generate Enc(RANDSUCI#1), Enc(RANDSUCI#2), etc.

Overview of Temporary SUCI:

Figure 7.2.1.Y.2-2: Temporary SUPI (overview)

The following steps are applicable:

1.) The UDM is creating the list of SUCI values and is encrypting these by using symmetric cryptography. (Rationale: Actually, symmetric cryptography is assumed to be quantum safe.) The UDM is creating a Key KSUPI for the SUPI encryption and must store this key, because this key is later used for the decryption.

2.) The list of encrypted SUCI’s is sent to the UE along with RANDSUPI. This RANDSUPI is used by UDM to generate the Key KSUPI (reference “Overview of Encrypted SUCI in UDM” of this document).

3.) The UE is storing the received list of encrypted SUCI’s. USIM/ ME will also use RANDSUPI to generate Key KSUPI.

4.) The UE is now selecting one encrypted SUCI.

5.)  The UE is sending the registration request to the UDM and is added new processed values into this message. The following needs to be processed by the UE: The root key is the Key K which is stored inside the USIM of that UE. The UE is creating a hash of that Key KSUPI. Furthermore, the UE is concatenating the Encrypted SUCI and is hashing both, the Encrypted SUCI together with the hashed Key KSUPI. The rationale for creating this concatenation is the following: The Encrypted SUCI is used by the UDM to verify the authentication of the SUCI value, while the hashed key KSUPI is used by the UDM to verify the authenticity of the UE (could also be called, the legitimacy of the UE for sending these information elements).

6.) The UDM is receiving the Registration Request message and is first using the Encrypted SUCI value for the look-up on which key is needed for the decryption. Now since the UDM knows which key is to be used and since it knows the UE, the UDM is taking the fetching the computed key KSUPI, is hashing this, and is fetching the encrypted SUCI from the local stored memory and is hashing the concatenated encrypted SUCI and the hashed key KSUPI. The outcome of this hashing (refers to the expected hash) will be compared with the received hash value.

7.) This refers to the registration and auth execution and completion. Rationale: The execution steps above refer basically to the auth of the encrypted SUCI and the authentication (legitimacy) of the UE. After this the normal Auth needs to be processed.

8.) After successful encrypted SUCI usage, both UE and UDM deletes this value from the list and same UE can’t use the same for further communications.

9.)10.)11.) This refers to the renewing and deployment of new list of encrypted SUCI values. UDM could use old RANDSUPI and continue to use the KSUPI for encryptions, but also UDM could decide to refresh this key KSUPI by creating new RANDSUPI and pass it to UE.

### 7.2.1.Y.3 	Evaluation

TBD

Editor’s Note: Further evaluation to be added.

* * * End of Changes * * * *