# 3GPP TSG-SA3 Meeting #125																	draft_S3-254139-r1

# Dallas, US, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.116 | CR | DraftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to TS 33.116 MME SCAS according to NESASG feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5GA |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change requests according to GSMA NESAS feedback, to ensure the SCAS is suitable for use in the NESAS scheme. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the reference, abbreviations, TC names, execution steps, expected format of evidence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.1, and 4.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	3GPP TR 33.117: "Catalogue of General Security Assurance Requirements".

[4]	3GPP TR 33.916: "Security assurance scheme for 3GPP network products for 3GPP network product classes".

[5]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[6]	void.

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 33.102: "3G security; Security architecture".

[9]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MME Application: The running processes (typically more than one) executing the software package for the MME functions and O&M functions of the MME network product class.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AUTS	AUThentication reSynchronisation token

EMM	EPS Mobility Management

GUTI	Globally Unique Temporary Identity

SRVCC	Single Radio Voice Call Continuity

TAU	Tracking Area Update

4	MME-specific security requirements and related test cases

4.1	Introduction

The structure of the present TS 33.116 is aligned with TS 33.117 [3] such that the MME-specific adaptation of a generic requirement can be always found in the present TS 33.116. Generic security requirements and test cases common to other network product classes have been captured in TS 33.117 [3] and are not repeated in the present document.

4.2	MME-specific adaptations of security functional requirements and related test cases

4.2.1	Introduction

4.2.2	Security functional requirements on the MME deriving from 3GPP specifications and related test cases

4.2.2.1	Security functional requirements on the MME deriving from 3GPP specifications – general approach

In addition to the requirements and test cases in TS 33.117, clause 4.2.2, an MME shall satisfy the following:

It is assumed for the purpose of the present SCAS that an MME conforms to all mandatory security-related provisions pertaining to an MME in:

-	3GPP TS 33.401: "EPS security architecture";

-	other 3GPP specifications that make reference to TS 33.401 or are referred to from TS 33.401 (e.g. TS 23.401 [7]).

Security procedures pertaining to an MME are typically embedded in mobility management procedures and are hence assumed to be tested together with them. Examples include:

-	AKA authentication is embedded in an Attach procedure or a TAU procedure.

-	Security Mode Control is embedded in an Attach procedure or a TAU procedure.

-	The derivation of a mapped security context is embedded in inter-RAT mobility procedures.

4.2.2.2	Authentication and key agreement procedure

4.2.2.2.1	Access with GSM SIM forbidden

Requirement Name: GSM SIM access forbidden

Requirement Reference: TS 33.401[5], clause 6.1.1

Requirement Description: "Access to E-UTRAN with a GSM SIM or a SIM application on a UICC shall not be granted." as specified in TS 33.401 [5], clause 6.1.1.

Threat References: TR 33.926 [9], clause A.2.2.1 Access to GSM.

Test Case:

Test Name: TC_GSM_SIM_FORBD_MME

Purpose:

Verify that access to EPS with a GSM SIM is not possible.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with HSS. HSS may be simulated.

Execution Steps

- The tester triggers the HSS to send an authentication data response with GSM authentication vector to the MME under test.

- The tester checks whether the MME under test rejects the UE authentication.

Expected Results:

MME rejects UE authentication when receiving GSM authentication vector from HSS.

NOTE: 	When both the MME and HSS function correctly, the GSM authentication vector is never included in the authentication data response from the HSS to the MME.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.2.2	Re-synchronization

Requirement Name: Inclusion of RAND, AUTS

Requirement Reference: TS 33.401[5], clause 6.1.2

Requirement Description: "In the case of a synchronization failure, the MME shall also include RAND and AUTS." as specified in TS 33.401 [5], clause 6.1.2.

Threat References: TR 33.926 [9], clause A.2.2.2 Resynchronization.

Test Case:

Test Name: TC_SYNC_FAIL_SEAF_MME

Purpose:

Verify that Re-synchronization procedure works correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with UE and HSS. UE and HSS may be simulated.

Execution Steps

- The tester triggers the UE to send an AUTHENTICATION FAILURE message with the EMM cause #21 "synch failure" and a re-synchronization token AUTS to the MME under test.

- The MME under test receives the AUTHENTICATION FAILURE message from the UE.

- The tester checks the authentication data request sent to the HSS by the MME under test.

-

Expected Results:

The MME includes the stored RAND and the received AUTS in the authentication data request to the HSS.

NOTE: 	When RAND and AUTS are not included in the authentication data request to the HSS then the HSS will return a new authentication vector (AV) based on its current value of the sequence number SQNHE (cf. TS 33.102 [8], clause 6.3.5) A new authentication procedure between MME and UE using this new AV will be successful just the same if the cause of the synchronisation failure was the sending of a "stale" challenge, i.e. one that the UE had seen before or deemed to be too old. But if the cause of the synchronisation failure was a problem with the sequence number SQNHE in the HSS (which should be very rare), and the RAND and AUTS are not included in the authentication data request to the HSS, then an update of SQNHE based on AUTS will not occur in the HSS, and the new authentication procedure between MME and UE using the new AV will fail again. This can be considered a security-relevant failure case as it may lead to a subscriber being shut out from the system permanently.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.2.3	Integrity check of Attach message

Requirement Name: Integrity check of Attach message

Requirement Reference: TS 33.401[5], clause 6.1.4

Requirement Description: "If the user cannot be identified or the integrity check fails, then the MME shall send a response indicating that the user identity cannot be retrieved." as specified in TS 33.401, clause 6.1.4.

Threat References: TR 33.926 [9], clause A.2.2.3 Failed Integrity check of Attach message.

Test Case:

Test Name: TC_INT_CHECK_MME

Purpose:

Verify that secure user identification by means of integrity check of Attach request works correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with new and old MME. New MME may be simulated.

Execution Steps

- The tester triggers the new MME to send an Identification Request message with incorrect integrity protection to the old MME under test.

- The old MME under test receives the Identification Request message from the new MME.

- The tester checks the response message sent by the old MME under test.

Expected Results:

The old MME sends a response indicating that the user identity cannot be retrieved.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.2.4	Not forwarding EPS authentication data to SGSN

Requirement Name: Not forwarding EPS authentication data to SGSN

Requirement Reference: TS 33.401[5], clause 6.1.4

Requirement Description: "EPS authentication data shall not be forwarded from an MME towards an SGSN." as specified in TS 33.401[5], clause 6.1.4.

Threat References: TR 33.926 [9], clause A.2.2.4 Forwarding EPS authentication data to SGSN.

Test Case:

Test Name: TC_NOT_FORWARD_EPS_AUTH_MME

Purpose:

Verify that EPS authentication data remains in the EPC.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with MME and SGSN. SGSN may be simulated.

Execution Steps

- The tester triggers the SGSN to send an Identification Request message to the MME under test.

- The MME under test receives the Identification Request message from SGSN.

- The tester checks the response message sent by the MME under test.

Expected Results:

The response to the SGSN does not include EPS authentication data.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.2.5	Not forwarding unused EPS authentication data between different security domains

Requirement Name: Not forwarding unused EPS authentication between different security domains

Requirement Reference: TS 33.401[5], clause 6.1.5

Requirement Description: "Unused EPS authentication vectors, or non-current EPS security contexts, shall not be distributed between MMEs belonging to different serving domains (PLMNs)." as specified in TS 33.401, clause 6.1.5.

Threat References: TR 33.926 [9], clause A.2.2.5 Forwarding unused EPS authentication data between different security domains.

Test Case:

Test Name: TC_NOT_FORWARD_UNUSED_EPS_AUTH_MME

Purpose:

Verify that unused EPS authentication data remains in the same serving domain.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with old and new MME in different serving domains. New MME may be simulated.

Execution Steps

- The tester triggers the new MME to send an Identification Request message to the old MME under test.

- The old MME under test receives the Identification Request message from the new MME.

- The tester checks the response message sent by the old MME under test.

Expected Results:

The response to the new MME does not include unused EPS authentication data.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.3	Security mode command procedure

4.2.2.3.1	Bidding down prevention

Requirement Name: Bidding down prevention

Requirement Reference: TS 33.401[5], clause 7.2

Requirement Description: "The SECURITY MODE COMMAND shall include the replayed security capabilities of the UE." as specified in TS 33.401[5], clause 7.2.

Threat References: TR 33.926 [9], A.2.3.1 Bidding Down.

Test Case:

Test Name: TC_BIDDING_DOWN_MME

Purpose:

Verify that bidding down by eliminating certain UE capabilities on the interface from UE to MME is not possible.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

- The tester triggers the UE to send an Attach request message includes security capabilities of the UE to the MME under test.

- The MME under test receives the Attach request message from the UE and responses with a SECURITY MODE COMMAND message.

- The tester checks the SECURITY MODE COMMAND message sent by the MME under test.

Expected Results:

MME includes the same security capabilities of the UE in the SECURITY MODE COMMAND message.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.3.2	NAS integrity algorithm selection and use

Requirement Name: NAS integrity algorithm selection

Requirement Reference: TS 33.401[5], clause 7.2.4.3.1

Requirement Description: "The MME shall protect the SECURITY MODE COMMAND message with the integrity algorithm, which has the highest priority according to the ordered lists." as specified in TS 33.401 [5], clause 7.2.4.3.1."

NOTE: 	The text in TS 33.401 [5], clause 7.2.4.3.1 is somewhat incomplete. It should properly read: "…which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities."

Threat References: TR 33.926 [9], A.2.3.2 NAS integrity selection and use

Test Case:

Test Name: TC_NAS_INT_SELECTION_USE_MME

Purpose:

Verify that NAS integrity protection algorithm is selected and applied correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

- The tester triggers the UE to send an Attach request message with Initial Attach type of the UE to the MME under test.

- The tester filters the Security Mode Command and Security Mode Complete messages.

- The tester checks the selected integrity algorithm in the SMC against the list of NAS integrity algorithm and the UE EPS security capabilities supported by the UE. The tester checks the MAC verification of the Security Mode Complete at the MME under test.

Expected Results:

1.	The MME has selected the integrity algorithm which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities. The MME checks the message authentication code on the SECURITY MODE COMPLETE message.

2.	The MAC in the SECURITY MODE COMPLETE is verified, and the NAS integrity protection algorithm is selected and applied correctly.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.3.3	NAS NULL integrity protection

Requirement Name: NAS NULL integrity protection

Requirement Reference: TS 33.401[5], clause 5.1.4.1

Requirement Description: "EIA0 shall only be used for unauthenticated emergency calls." as specified in TS 33.401[5], clause 5.1.4.1."

Threat References: TR 33.926 [9], A.2.3.3 NAS NULL integrity protection

Test Case:

Test Name: TC_NAS_NULL_INT_MME

Purpose:

Verify that NAS NULL integrity protection algorithm is used correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

- The tester triggers the UE to initiate a non-emergency attach procedure.

- The MME under test selects integrity algorithm after successful UE authentication.

- The MME under test sends the SECURITY MODE COMMAND message containing the selected integrity algorithm.

- The tester checks the SECURITY MODE COMMAND message.

Expected Results:

The selected integrity algorithm is different from EIA0.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.3.4	NAS confidentiality protection

Requirement Name: NAS confidentiality protection

Requirement Reference: TS 33.401[5], clause 7.2.4.3.1

Requirement Description: "The UE…sends the NAS security mode complete message to MME ciphered and integrity protected." as specified in TS 33.401[5], clause 7.2.4.3.1.

Threat References: TR 33.926 [9], A.2.3.4 NAS confidentiality protection

Test Case:

Test Name: TC_NAS_CONFIDENTIALITY_MME

Purpose:

Verify that NAS confidentiality protection algorithm is applied correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps

- The tester triggers the UE to send a SECURITY MODE COMPLETE message without confidentiality protection.

- The MME under test receives the SECURITY MODE COMPLETE message from the UE.

- The tester checks the selected confidentiality algorithm selected and checks whether the MME rejects the message.

Expected Results:

If a confidentiality algorithm different from EEA0 was selected the MME rejects the message.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.4	Security in intra-RAT mobility

4.2.2.4.1	Bidding down prevention in X2-handovers

Requirement Name: Bidding down prevention in X2-handovers

Requirement Reference: TS 33.401[5], clause 7.2.4.2.2

Requirement Description: "The MME shall verify that the UE EPS security capabilities received from the eNB are the same as the UE EPS security capabilities that the MME has stored." as specified in TS 33.401[5], clause 7.2.4.2.2."

Threat References: TR 33.926 [9], A.2.4.1 Bidding down on X2-Handover

Test Case:

Test Name: TC_BIDDING_DOWN_X2_MME

Purpose:

Verify that bidding down is prevented in X2-handovers.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with (target) eNB. eNB may be simulated.

The MME is configured to log the event of a UE EPS security capability mismatch.

Execution Steps

- The tester sends EPS security capabilities for the UE, different from the ones stored in the MME, to the MME under test using a Path-Switch message.

- The tester captures the Path-Switch Acknowledge message sent by MME under test to the target eNB.

- The tester checks the MME log regarding the capability mismatch.

Expected Results:

The MME logs the event.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.4.2	NAS integrity protection algorithm selection in MME change

Requirement Name: NAS integrity protection algorithm selection in MME change

Requirement Reference: TS 33.401[5], clause 7.2.4.3.2

Requirement Description: "In case there is change of MMEs and algorithms to be used for NAS, the target MME shall initiate a NAS security mode command procedure and include the chosen algorithms and the UE security capabilities (to detect modification of the UE security capabilities by an attacker) in the message to the UE (see clause 7.2.4.4). The MME shall select the NAS algorithms which have the highest priority according to the ordered lists (see clause 7.2.4.3.1)." as specified in TS 33.401[5], clause 7.2.4.3.2."

Threat References: TR 33.926 [9], A.2.4.2 NAS integrity protection algorithm selection in MME change

Test Case:

Test Name: TC_NAS_INTEGRITY_IN_MME_CHANGE

Purpose:

Verify that NAS integrity protection algorithm is selected correctly.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with source and target MME. Source MME may be simulated.

Execution Steps

- The tester triggers the source MME to send the UE EPS security capabilities and the NAS algorithms used by the source MME to the target MME under test over the S10 interface.

- The target MME under test selects the NAS algorithms which have the highest priority according to the ordered lists. The lists are assumed such that the algorithms selected by the target MME are different from the ones received from the source MME.

- The tester checks whether the target MME selects the NAS integrity protection algorithm correctly.

Expected Results:

The target MME initiates a NAS security mode command procedure and include the chosen algorithms and the UE security capabilities.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.5	Security in inter-RAT mobility

4.2.2.5.1	No access with GSM SIM via idle mode mobility

Requirement Name: Idle mode mobility into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 9.1.2

Requirement Description: "In case the MM context in the Context Response/SGSN Context Response indicates GSM security mode, the MME shall abort the procedure." as specified in TS 33.401, clause 9.1.2.

Threat References: TR 33.926 [9], A.2.5.1 GSM SIM access via idle mode mobility

Test Case:

Test Name: TC_NOT_ACCESS_GSM_SIM_IDLE_MME

Purpose:

Verify that GSM subscribers cannot obtain service in EPS via idle mode mobility.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with source SGSN and target MME. Source SGSN may be simulated.

Execution Steps:

- The tester triggers the source SGSN to send the MM context in the Context Response indicating GSM security mode to the target MME under test.

- The target MME under test receives the Context Response.

- The tester checks the response message sent by the MME under test.

Expected Results:

The MME aborts the procedure by acknowledging the Context Response from the SGSN with an appropriate failure cause.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.5.2	No access with GSM SIM via handover

Requirement Name: Handover into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 9.2.2

Requirement Description: "In case the MM context in the Forward relocation request message indicates GSM security mode (i.e. it contains a Kc), the MME shall abort the non-emergency call procedure." as specified in TS 33.401, clause 9.2.2.

Threat References: TR 33.926 [9], A.2.5.2 GSM SIM access via handover

Test Case:

Test Name: TC_NOT_ACCESS_GSM_SIM_HANDOVER_MME

Purpose:

Verify that GSM subscribers cannot obtain service in EPS via handovers.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with source SGSN and target MME. Source SGSN may be simulated.

Execution Steps:

- The tester triggers the source SGSN to send the MM context in the Forward Location Request message indicating GSM security mode to the target MME under test.

- The target MME under test receives the Forward Location Request message.

- The tester checks the response message sent by the MME under test.

Expected Results:

The MME aborts the procedure by responding to the Forward Relocation Request from the SGSN with an appropriate failure cause.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.5.3	No access with GSM SIM via SRVCC

Requirement Name: SRVCC into E-UTRAN forbidden for GSM subscribers

Requirement Reference: TS 33.401[5], clause 14.3.1

Requirement Description: "If the MME receives a GPRS Kc' from the source MSC server enhanced for SRVCC in the CS to PS HO request, the MME shall reject the request." as specified in TS 33.401, clause 14.3.1.

Threat References: TR 33.926 [9], A.2.5.3 GSM SIM access via SRVCC

Test Case:

Test Name: TC_NOT_ACCESS_GSM_SIM_SRVCC_MME

Purpose:

Verify that GSM subscribers cannot obtain service in EPS via SRVCC into E-UTRAN.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with source MSC server and target MME. Source MSC server may be simulated.

Execution Steps

- The tester triggers the source MSC to send the GPRS Kc' and the CKSN'PS in the CS to PS handover request to the target MME under test.

- The target MME under test receives the CS to PS handover request.

- The tester checks whether the MME under test rejects the request.

Expected Results:

The MME rejects the request.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.2.6	Security Aspects of IMS Emergency Session Handling

4.2.2.6.1	Authentication failure for emergency bearers

NOTE: 	The use of NULL integrity is addressed in clause D.3.2.3.3.

Requirement Name: Emergency bearer establishment when authentication fails

Requirement Reference: TS 33.401 [5], clause 15.1.

Requirement Description: "The MME or UE shall always release any established non-emergency bearers, when the authentication fails in the UE or in the MME." as specified in TS 33.401, clause 15.1.

Threat References: TR 33.926 [9], A.2.6	Threats related to release of non-emergency bearer

Test Case:

Test Name: TC_AUTH_FAIL_EMER_MME

Purpose:

Ensure that the MME enforces that only emergency bearers can be used without successful authentication.

Procedure and Execution Steps:

Pre-Conditions:

Test environment with MME and UE. UE may be simulated. The serving network policy allows unauthenticated IMS Emergency Sessions.

Execution Steps

- The tester triggers the UE to send the initial attach request for EPS emergency bearer services to the target MME under test.

- The MME under test initiates an authentication, which fails.

- The UE attached for EPS emergency bearer services sends the PDN Connectivity request for EPS non-emergency bearer services.

- The tester checks whether the MME under test acts correctly.

Expected Results:

The MME allows to continue the set up of the emergency bearer, and rejects the PDN Connectivity request for EPS non-emergency bearer services.

Expected Format of Evidence:

Evidence suitable for the interface, e.g., Screenshot, packet capture or application logs containing the operational results.

4.2.3	Technical Baseline

4.2.3.1	Introduction

This clause provides baseline technical requirements.

4.2.3.2	Protecting data and information

4.2.3.2.1	Protecting data and information – general

There are no MME-specific additions to clause 4.2.3.2.1 of TS 33.117.

4.2.3.2.2	Protecting data and information – unauthorized viewing

There are no MME-specific additions to clause 4.2.3.2.2 of TS 33.117.

4.2.3.2.3	Protecting data and information in storage

There are no MME-specific additions to clause 4.2.3.2.3 of TS 33.117.

4.2.3.2.4	Protecting data and information in transfer

There are no MME-specific additions to clause 4.2.3.2.4 of TS 33.117:

4.2.3.2.5	Logging access to personal data

There are no MME-specific additions to clause 4.2.3.2.5 of TS 33.117.

4.2.3.3	Protecting availability and integrity

There are no MME-specific additions to clause 4.2.3.3 of TS 33.117.

4.2.3.4	Authentication and authorization

There are no MME-specific additions to clause 4.2.3.4.

4.2.3.5	Protecting sessions

There are no MME-specific additions to clause 4.2.3.5 of TS 33.117.

4.2.3.6	Logging

All text from TS 33.117, clause 4.2.3.6 also applies to MMEs. There are no MME-specific adaptations or additions to this text.

4.2.4	Operating Systems

There are no MME-specific additions to clause 4.2.4 of TS 33.117.

4.2.5	Web Servers

There are no MME-specific additions to clause 4.2.5 of TS 33.117.

4.2.6	Network Devices

All text from TS 33.117, clause 4.2.6 also applies to MMEs. There are no MME-specific adaptations or additions to this text.

4.3	MME-specific adaptations of hardening requirements and related test cases

4.3.1	Introduction

4.3.2	Technical Baseline

All text from TS 33.117, clause 4.3.2 also applies to MMEs. There are no MME-specific adaptations or additions to this text.

4.3.3	Operating Systems

There are no MME-specific additions to clause 4.3.3 of TS 33.117.

4.3.4	Web Servers

There are no MME-specific additions to clause 4.3.4 of TS 33.117.

4.3.5	Network Devices

There are no MME-specific additions to clause 4.3.5 of TS 33.117.

4.4	MME-specific adaptations of basic vulnerability testing requirements and related test cases

All text from TS 33.117, clause 4.4 also applies to MMEs. There are no MME-specific adaptations or additions to this text.