3GPP TSG-SA3 Meeting #125	draft_S3-254147-r1

# Dallas, US, 17th - 21th November 2025

Source:	ZTE

Title:	Add evaluation for Solution #1.1

Document for:	Approval

Agenda item:	5.2.7

Spec:	3GPP TR 33.777

Version:	0.2.0

Work Item:	FS_Sensing_SEC

Comments

This contribution proposes to add evaluation for solution #1.1.

Proposed Changes

* * * First Change * * * *

6.1.1	Solution #1.1: Authorization for sensing service request from AF

6.1.1.1	Introduction

This solution addresses Key Issue #1: Security of authorization for sensing service invocation and revocation.

In this solution, the sensing service consumer is assumed to be an external AF. The NEF performs the access authorization by verifying the AF's identity, and the SF performs the service authorization by validating the feasibility and policy compliance of the specific sensing request parameters against network capabilities and operator rules.

Editor’s Note: Whether this solution is applicable only for external AF is FFS.

6.1.1.2	Solution details

This solution proposes mutual certificate-based authentication between the NEF and the external AF/sensing service consumer using TLS. Certificate based authentication follows the profiles given in 3GPP TS 33.310 [6], clause 6.1.3a. The identities in the end entity certificates is used for authentication and policy checks.

For the protection of communication between AF/sensing service consumer and NEF, TLS is used to provide integrity protection, replay protection and confidentiality protection for the interface between the NEF and the AF/sensing service consumer. Security profiles for TLS implementation and usage follow the provisions given in clause 6.2 of TS 33.210 [7].

After the authentication, the following procedures are used for authorizing sensing service request.

Figure 6.1.1.2-1: Procedure for sensing service authorization

- The AF sends sensing service request message to the NEF. The message includes AF ID, OAuth 2.0 token, and sensing service related parameters (e.g., target sensing area, sensing time, sensing accuracy, etc).

Editor’s note: Details of the sensing service related parameters are FFS.

- NEF performs the authorization check for the sensing service request. This includes:

- validating the OAuth 2.0 token presented by the AF; and

- checking the AF's subscription profile to verify that the AF is entitled to request the sensing service.

If the check fails, the NEF rejects the request with a failure cause.

- If the AF is authorized by the NEF to request for sensing service, the NEF discovers and selects the SF, and sends the Sensing service request message to the SF. This message includes sensing service related parameters.

- The SF performs sensing service authorization based on the sensing service related parameters. Specifically, this includes:

- validating the sensing service related parameters against operator-defined service policies (e.g., restricted zones, restricted time); and

- checking if the network has available resources to fulfill the request.

If the authorization fails, the SF rejects the request with a failure cause. The reject message is sent to AF via NEF.

Editor’s note: Other validations performed by SF for sensing service authorization are FFS.

- After successful authorization, the SF proceeds to execute the sensing service.

6-7. The SF provides sensing results in sensing service response to the AF via NEF.

Editor’s note: Details of the procedures are to be aligned with TR 23.700-14 [2].

6.1.1.3	Evaluation

This solution is based on the assumption that the sensing service consumer is an external AF. This solution does not address authorization for internal AF.

This solution proposes to reuse the existing mechanism to perform mutual authentication and secure communication between sensing service consumer and NEF. Details of the OAuth framework is not addressed in this solution.

The following impacts are needed:

-	NEF needs to perform the access authorization by verifying the AF's identity;

-	SF needs to perform sensing service authorization.

* * * End of Changes * * * *