3GPP TSG-SA3 Meeting #125	S3-254154

# Dallas, US, 17th - 21th November 2025

Source:	ZTE

Title:	Resolve EN in Solution #1

Document for:	Approval

Agenda item:	5.2.9

Spec:	3GPP TR 33.700-30

Version:	0.1.0

Work Item:	FS_5GSAT_Ph4_SEC

Comments

This contribution proposes to update details of Sol #1 to address the editor’s notes.

Proposed Changes

* * * First Change * * * *

6.1	Solution #1: Derivation of Satellite-Specific NAS keys for S&F Operation

6.1.1	Introduction

This solution addresses Key Issue #1: Authenticated UE to exchange NAS messages with multiple satellites in split-MME architecture.

This solution proposes a mechanism to derive unique NAS integrity and encryption keys for each satellite by using the satellite ID as an additional input parameter during the NAS key derivation.

6.1.2	Solution details

In this solution, it is proposed to derive distinct set of NAS keys for each satellite from the common root key KASME. The satellite-specific NAS keys are derived by the UE and the network using the KDF as specified in TS 33.220 [x].

For a serving Satellite n, the NAS integrity key KNASint and the NAS encryption key KNASenc are derived from the KASME with the following parameters as input:

-	FC = 0xxx

-	P0 = algorithm type distinguisher

-	L0 = length of algorithm type distinguisher (i.e. 0x00 0x01)

-	P1 = algorithm identity

-	L1 = length of algorithm identity (i.e. 0x00 0x01)

-  P2 = Satellite ID n.

-  L2: length of Satellite ID n.

Where Satellite ID is an identifier uniquely indicating an MME-onboard. The Satellite ID of a given satellite is broadcast by the eNB within the SIB31 and the Satellite ID of the satellites that might be serving a given UE are included within the S&F Monitoring List, which is sent by the MME to indicate the satellite(s) that the UE may (re)-attempt NAS procedures (TS 23.401 clause 4.13.9.1).

As a result of using satellite-specific keys, the UE and each MME-onboard maintain independent pairs of NAS COUNT for their mutual communication. The NAS COUNTs are not synchronized with other satellites.

Upon receiving the UE context, the MME-onboard derives its satellite-specific NAS keys using its own Satellite ID. The MME-onboard can provide the UE an indicator, indicating that the separate NAS keys are implemented, together with the monitoring list. The UE derives the NAS keys for a Satellite ID immediately upon receiving the S&F Monitoring List, or when it determines to initiate a NAS procedure with a satellite present in the list with which no prior keys have been derived.

The UE and the MME-ground need to manage the multiple satellite-specific security context by associating each Satellite ID with the satellite-specific NAS keys and NAS COUNTs. When the MME-ground detects that the NAS COUNT for any satellite is about to wrap around, the MME-ground performs AKA procedure with the UE via any MME-onboard. Upon successful AKA completion, a new KASME is established. The MME-grounds then provide the new KASME to all MME-onboards in the S&F Monitoring List. The MME-onboards and the UE then derive new satellite-specific NAS keys and reset NAS COUNTs.

Editor’s Note: The establishment of AS security in this solution is FFS.

* * * End of Changes * * * *