# 3GPP TSG-SA3 Meeting #125																			S3-254179-r1

# Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on updating clause 6 for TLS 1.2 and TLS 1.3

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.2.0

Work Item:

Comments

This contribution proposes text changes for subclause of TLS1.2 and TLS 1.3 protocols. The main changes are summarized as follows:

- Added description for EAP-TTLS and OAuth 2.0 so that all protocols listed in TR 33.938 are covered

- An acronym is added.

- Corrections to clause 6.7.1 and 6.7.2 for alignment with text in clauses 4.1 and 4.2

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ANSSI			Agence Nationale de la Sécurité des Systèmes d'Information

CA			Certification Authority

CBOR			Concise Binary Object Representation

COSE			CBOR Object Signing and Encryption

CRL			Certificate Revocation Lists

CRQC			Cryptographically Relevant Quantum Computer

DSA			Digital Signature Algorithm

ECC			Elliptic Curve Cryptography

ECDH			Elliptic Curve Diffie–Hellman key Exchange

ECIES			Elliptic Curve Integrated Encryption Scheme

FN-DSA			Fast-Fourier Transform over NTRU-Lattice-Based DSA

HBS			Hash-Based Signature

HQC			Hamming Quasi-Cyclic

HSS			Hierarchical Signature System

IKEv2			Internet Key Exchange Protocol Version 2

JSON			JavaScript Object Notation

JWE			JSON Web Encryption

JWS			JSON Web Signature

KEM			Key Encapsulation Mechanism

LTS			Long Term Stable

MIKEY-SAKKE			Multimedia Internet KEYing – Sakai-Kasahara Key Encryption

ML-DSA			Module-Lattice-Based DSA

ML-KEM			Module Lattice-Based Key-Encapsulation Mechanism

NCSC 			National Cyber Security Centre

NSA			National Security Agency

NSM			National Security Memorandum

NTRU			Nth-degree Truncated Polynomial Ring Units

PKI			Public Key Infrastructure

PKIX			Public Key Infrastructure X.509

PQC			Post-Quantum Cryptography

SA			Security Association

SDO			Standards Development Organizations

SECG			Security Engineering & Consulting Group

SLH-DSA			Stateless Hash-Based DSA

SUCI			Subscription Concealed Identifier

TLS 1.2			Transport Layer Security Version 1.2

TLS 1.3			Transport Layer Security Version 1.3

XMSS			eXtended Merkle Signature Scheme

* * * Next Change * * * *

## 6.6	TLS 1.2

### 6.6.1	General

The TLS 1.2 handshake in IETF RFC 5246 [57] is used in TLS 1.2, DTLS 1.2, EAP-TLS 1.2, EAP-TTLS, and OAuth 2.0. The DTLS handshake is also applied in DTLS over SCTP and can be used in DTLS-SRTP.

The 3GPP TLS profile is defined in clause 6.2 of 3GPP TS 33.210 [59]. Since Release 15, TLS 1.3 has been mandatory for all 3GPP core network nodes, and from Release 16 onward, it is mandatory for all nodes. Because TLS always negotiates the highest mutually supported version, any use of TLS 1.2 in a 3GPP system from Rel-16 onward implies that at least one node is non-compliant with 3GPP specifications.

While a fully updated TLS 1.2 implementation could theoretically provide strong security against classical adversaries in scenarios where identity protection is not required, in practice, TLS 1.2 is only negotiated by outdated implementations. These often suffer from one or more known vulnerabilities.

Therefore, TLS 1.2 is expected to already have been fully phased out in 5G systems.

### 6.6.2	Current Work in IETF

TLS 1.2 has been obsoleted since 2018, as superseded by TLS 1.3 in IETF RFC 8446 [58]. The IETF will no longer approve any additions or updates to TLS 1.2, including PQC support (IETF draft-ietf-tls-tls12-frozen-08 [60]).

### 6.6.3		3GPP Considerations

* * * Next Change * * * *

## 6.7	TLS 1.3

### 6.7.1	General

The TLS 1.3 handshake protocol as defined in clause 4 of IETF RFC 8446 [58] is used in TLS 1.3, EAP-TLS 1.3, EAP-TTLS 1.3, OAuth 2.0, DTLS 1.3, and QUIC, and it can also be used in DTLS-SRTP. Since Release 15, TLS 1.3 has been mandatory to implement for the core network (cf. Annex E in TS 33.310 v15.0.0), and starting in Release 16, it has been mandatory to implement also for the ME (cf. Annex E in TS 33.310 v16.0.0).

IETF is in general recommending hybridization of KEMs and the hybrid KEM X25519MLKEM768 [65] has already received widespread implementation support and is the default in OpenSSL. It has been reported [25] that over 40% of all HTTPS client requests now use X25519MLKEM768. Standalone ML-KEM [64], ML-DSA [66] have seen more limited implementation but are supported in OpenSSL 3.5 LTS.

### 6.7.2	Current Work in IETF

The IETF has prioritized post-quantum migration in TLS as follows[61]:

•	Now (Hybrid + Pure ML-KEM)

•	Later (signatures)

•	Much later (dual certificates/composite signatures)

The IETF TLS Working Group has planned not to adopt work on hybrid signatures until "much later" [61].

The IETF TLS Working Group has introduced multiple drafts to enable a smooth transition to PQC in TLS 1.3. These proposals address both key exchange and authentication. These mechanisms collectively aim to maintain interoperability, minimize latency, and provide quantum-resistant security during and after the PQC transition.

In an LS to GSMA [62], the IETF TLS Working Group stated that they believe the IETF Adopted Draft "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3" [65] is stable enough to be used as normative reference, and that referencing an adopted draft normatively is a practice that other organizations follow as well.

#### 6.7.2.1	IETF RFCs

No RFCs for the usage of PQC algorithms in TLS 1.3 are published yet.

Editor's Note: several of the adopted drafts are in the final stages and may be published before this document is finalised.

#### 6.7.2.2	IETF Adopted Drafts

-	draft-ietf-tls-hybrid-design-16, "Hybrid key exchange in TLS 1.3" [63], specifies combining multiple key exchange algorithms (e.g., classical ECDHE with a PQ KEM) so that session security holds if at least one component remains secure.

-	draft-ietf-tls-mlkem-04, "ML-KEM Post-Quantum Key Agreement for TLS 1.3" [64], proposes to use the NIST specified ML-KEM [21] in TLS 1.3.

-	draft-ietf-tls-mldsa-00, "Use of ML-DSA in TLS 1.3" [66], proposes to use the NIST specified ML-DSA [22] in TLS 1.3.

-	draft-ietf-tls-ecdhe-mlkem-00, "Post-quantum hybrid ECDHE-MLKEM Key Agreement for TLSv1.3" [65], defines three hybrid key agreements for TLS 1.3: X25519MLKEM768, SecP256r1MLKEM768, and SecP384r1MLKEM1024.

### 6.7.3		3GPP Considerations

Editor’s Note: This clause does not include any conclusions.

* * * End of Changes * * * *