3GPP TSG-SA3 Meeting #125	S3-254271

Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon

Title:	Solution on the AEAD interface

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.1.0

Work Item:	FS_AEAD

Comments

Key issue#2 on algorithm interface is addressed by this solution. The input and output parameters are clarified.

* * * First Change * * * *

## 5.Y	Solution #Y: Interface of AEAD

### 5.Y.1	Introduction

This solution is proposed to address the key issue#2 on AEAD interface.

### 5.Y.2	Solution details

The input parameters to the AEAD algorithm include:

- a 256-bit AEAD key named KEY,

- a 48-bit EXTRA_IV,

- a 32-bit COUNT,

- a 5-bit bearer identity BEARER,

- the 1-bit direction of the transmission i.e. DIRECTION. The DIRECTION bit shall be 0 for uplink and 1 for downlink.

- 1-bit MODE. The MODE bit shall be 0 for encryption and 1 for decryption.

- 5-bit MAC_BYTES,

- the length of the Additional Authenticated Data (AAD) required i.e. 32-bit AAD_LENGTH,

- and the length of the Input Bit Stream(IBS) required i.e. 32-bit S_LENGTH.

Figure 5.Y.2-1: Interface of AEAD algorithm

Based on the input parameters, the output ciphertext (Output Bit Stream) is generated. Based on these input parameters the sender computes a message authentication code (MAC-I/NAS-MAC). The message authentication code is then appended to the message when sent. The receiver computes the expected message authentication code XMAC-I/XNAS-MAC.

### 5.Y.3	Evaluation

Key issue#2 on algorithm interface is addressed by this solution. The input and output parameters are clarified.

* * * End of 1st Change * * * *