# 3GPP TSG-SA3 Meeting #125																			S3-254342-r1

# Dallas, US, 17 – 21 November 2025

Source:	LG Electronics

Title:	EN Resolution for solution #5

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	v0.2.0

Work Item:	FS_CryptoPQC

Comments

This proposal updates solution #5 to resolve Editor’s notes.

* * * First Change * * * *

7.2.1.5	Solution #5 to SUCI calculation: Enhancement on SUCI calculations using quantum key

7.2.1.5.1	Introduction

This solution provides enhancement for SUCI calculations based on existing ECIES scheme.

7.2.1.5.2	Solution details

This solution describes SUCI calculations using Quantum communication interface . The solution requires two distinct interfaces between the User Equipment (UE) and the Home Network (HN): one for the 5G/6G air interface (e.g., the N1 interface in 5G) and another for quantum communication interface.

The Home Network randomly generates Quantum Public/Private keys which is in a quantum state for the UE. The UE can provision the Public key of HN and the Quantum Public key. Based on ECIES scheme, the ephemeral public key, cipher text, and MAC tag can be generated as an output. Additionally, using the Quantum Public key, the cipher text can be transformed into data stream which is in a quantum state. The cipher text in a quantum state is delivered to the Home Network via Quantum communication interface. The Home Network transforms it into data stream with the Quantum Private key, then decrypts the ciphered text and verifies MAC.

7.2.1.5.2.1	Processing on UE side

The steps shown Figure 7.2.X.Y.2.1 are described as below:

- As a prerequisite, the UE provisions both Public key of HN and Quantum Public key. The Home Network generates the Quantum Public key and Quantum Private key in quantum a state.

- The UE generates Ephemeral key pair consisting of Ephemeral Public Key and Ephemeral Private Key.

- Based on the generated Ephemeral Private Key and the Public key of Home Network, the UE generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Encryption Key, ICB and Ephemeral MAC Key are generated.

- Plaintext is ciphered using the Ephemeral Encryption Key.

- The ciphered text and the Ephemeral MAC key are used to create MAC-tag value.

- The cipher text value is transformed into data stream which is in a quantum state after running Unitary operation (e.g., Qubit rotation, Pauli operation) with the Quantum Public key.

Figure 7.2.1.5.2.1: Encryption at UE

The final output consists of two types:

Concatenation of the ECC ephemeral public key, the MAC tag value, and any additional parameters.

Quantum ciphertext in a quantum state.

The first output is transmitted to the Home Network via the 5G/6G air interface, while the second output is delivered via the quantum communication interface.

7.2.1.5.2.2	Processing on home network side

The steps shown Figure 7.2.1.5.2.2 are described as below:

- The cipher text value in a quantum state is transformed into data stream (bit stream) after running Unitary operation (e.g., Qubit rotation, Pauli operation) with the Quantum Private key.

- Based on the received Ephemeral Public Key, the Home Network generates Ephemeral Shared Key.

- Using ECIES scheme, Ephemeral Decryption Key, ICB and Ephemeral MAC Key are generated.

- The ciphered text is deciphered using the Ephemeral Decryption Key.

- The Home Network verifies received MAC.

Figure 7.2.1.5.2.2: Decryption at Home Network

Editor’s Note: Whether and how to support hybrid encryption.

Editor's Note: The definition and usefulness of these terms for SUCI calculation are FFS: Post-quantum threat, Quantum channel, Quantum public key, Quantum-encapsulated, Quantum-encrypted, Quantum-ciphered, Quantum Private key.

Editor’s Note: How to use standardized algorithm is FFS.

* * * End of Changes * * * *