3GPP TSG-SA3 Meeting #124	draft_S3-254390-r2(+S3-254236+S3-254235)

# Wuhan, China, 13 – 17 October 2025

Source:	Xiaomi, InterDigital, OPPO

Title:	New Security Area on Sensing Security and Privacy

Document for:	Approval

Agenda item:	5.3.1

Spec:	3GPP TR 33.801-01

Version:	0.1.0

Work Item:	FS_6G_SEC

Comments

For the study of 6G architecture, the study on integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data and signal is one of the work tasks.

In TR 38.914 for the study on 6G scenarios and requirements, it is documented that the 6GR and 6G RAN architecture should at least support use cases of detection and/or tracking of passive objects, including at least UAVs, human, vehicles and AGVs. In the moderator's summary for RAN-led 6G study, it was endorsed that 6GR should study different sensing modes.

According to TR 33.777, the scope of 5GA sensing security only focuses on the security aspects of gNB-based mono-static sensing mode for UAV sensing target use cases. It means that the sensing objects (including human, vehicles and AVG) agreed in TR 38.914 and the sensing modes endorsed for RAN-led 6G study which are not in the scope of TR 33.777 need to be studied in 6G system to support the RAN-led 6G study and SA2-led 6G architecture study.

Based on above analysis, this contribution proposes a new security area on sensing security and privacy for TR 33.801-01.

* * * First Change * * * *

4	Security areas and high level security requirements

4.1	Security areas

Editor's Note: This clause further clarifies the scope of the study by listing the security areas that SA3 is working on.

This document includes the following security areas:

- UE to Core Network Security deals with the UE to Core Network communication security. e.g., management of UE and network NAS security contexts, the associated key hierarchy, key derivation and key usage in the 6G System.

- RAN security deals with the security aspects of 3GPP access network, e.g., RAN architecture, protocol stack, interfaces, procedures, interaction with UEs.

- Subscription Authentication and Authorization deals with different aspects of access-agnostic authentication, authorization and related privacy aspects (i.e. subscriber identifier privacy) for UEs accessing 6G network

X	Sensing security and privacy deals with the security and privacy protection for integration of sensing and communication considering the use cases in TR 38.914 [2], the sensing modes in the Study on 6G Radio, and multiple sources of sensing data and signal supported in TR 23.801-01 [4].

* * * Next Change * * * *

5	Key issues and solutions

5.x	Security area #x: Sensing security and privacy

5.x.1	Introduction

TR 33.777 [x] of 5GA sensing security only focuses on the security aspects of gNB-based sensing for aerial objects. According to TR 38.914 [2], 6GR and 6G RAN architecture should support use cases of detection and/or tracking of passive objects (e.g. UAV, human, vehicle and AGV). In the Study on 6G Radio, more sensing modes than gNB-based mode will be studied. In addition, integration of Sensing and Communication considering more sensing modes and other sources of sensing data and signal (e.g. non-3GPP sensing data) is also one of the work tasks of the Study on Architecture for 6G system.

This security area considers how 6G system provides security and privacy protection for integration of Sensing and Communication with more use cases agreed in TR 38.914 [2] and TR 23.801-01 [4], different sensing modes supported in the Study on 6G Radio, and multiple sources of sensing data and signal introduced in TR 23.801-01 [4], which are not in the scope of TR 33.777 [x].

Editor's Note: more aspects are FFS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]		3GPP TR 33.777: "Study on Security and Privacy Aspects of Integrated Sensing and Communication".

* * * End of Changes * * * *