3GPP TSG-SA3 Meeting #125	draft_S3-254553-r1

Dallas, US, 17 – 21 November 2025														revision of S3-254268

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0061 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resloving the editor’s note on randomness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is an EN on the parameter randomness and related definition. The proposal is to resolve it by adding a reference example. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification is provided to address the editor’s note on randomness. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsolved editor’s note. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Removed the recommendation and related references and included some statistics. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

-

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n. AIOTF shall retrieve RANDAIOT_n from ADM.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

3. NG-RAN shall include RANDAIOT_n in  the paging request message to the AIoT device in addition to other device identification information.

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

NOTE 2: While a legitimate network is performing an inventory operation, an attacker may cause amplification of resource exhaustion at the legitimate network side by sending AIoT paging messages for all devices or to a large group of devices, which causes large number of devices sending D2R messages to the legitimate network that the legitimate network does not expect to receive. The security measure to such amplification of resource exhaustion attack is not specified in present document.

4. Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d, calculate RESAIOT using KAIoT_root and RANDAIOT_n (see Annex A.2) for network authenticating AIoT Device.

NOTE X: The generation of RANDAIOT_d needs to be random. For a cryptographically secure pseudorandom generator, the probability of a draw of a 128 bit number is  close to 1/2128 and the entropy is 128 bits.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identification information, RANDAIOT_n  and RANDAIOT_d to ADM.

NOTE 3: the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), which has load impact to ADM.

8. ADM shall calculate XRESAIOT using the same method as in AIoT device (see Annex A.2).

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, for command case, AIOTF shall acquire KAIoTF from ADM. ADM shall calculate KAIoTF if receiving request from AIOTF (see AnnexA.2). ADM sends KAIoTF to AIOTF.

The steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

For the command procedure, the AIoT device implicitly authenticates the network via the verification of MAC which is derived using the KCommand_int as specified in clause 5.3.2 of present document.

* * * End of Changes * * * *