# 3GPP TSG-SA3 Meeting #125																				S3-254556-r2

# Dallas, US, 17 – 21 November 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.369 | CR | 0065 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of key lengths and length of RE/XRES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Rel AmbientIoT-SEC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The key lengths and the RES/XRES lengths are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RESAIOT/XRESAIOT , KCommand_enc / KCommand_int are 128 Bits each. KAIOTF , KAIOT_root are 256 Bits each |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.2, A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Begin of Changes

# A.2	RESAIOT and XRESAIOT derivation function

When deriving a RESAIOT (of variable length between 0x00 0x04 and 0x00  0x10 bytes) and XRESAIOT (of variable length between 0x00 0x04 and 0x00  0x10 bytes) from KAIOT_root (of at least 128 bit length), the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = RANDAIOT_d.

-	L1 = length of RANDAIOT_d (i.e. 0x00  0x10),

-	P2 = AIoT device permanent identifier,

-	L2 = length of AIoT device permanent identifier,

The input key KEY shall be KAIOT_root.

The RESAIOT/XRESAIOT is identified with the 128 least significant bits of the output of the KDF.

# A.3	KAIOTF derivation function

When deriving a KAIOTF from KAIOT_root, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = RANDAIOT_d.

-	L1 = length of RANDAIOT_d (i.e. 0x00  0x10),

The input key KEY shall be the KAIOT_root.

The KAIOT is identified with the 128 least significant bits of the output of the KDF.

# A.4	KCommand_enc and KCommand_int derivation function

When deriving a KCommand_enc or KCommand_int from KAIOTF, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = algorithm identity as specified in TS 33.501[5].

-	L0 = length of algorithm identity (i.e. 0x00 0x01)

The input key KEY shall be the KAIOTF.

The KCommand_enc / KCommand_int is identified with the 128 least significant bits of the output of the KDF.

End of Changes