3GPP TSG-SA3 Meeting #125	draft_S3-254582-r1

Dallas, US, 17 – 21 November 2025 		revision of S3-254038

Source:	Interdigital

Title:	Update Solution#2

Document for:	Approval

Agenda item:	5.2.6

Spec:	3GPP TR 33.785

Version:	0.2.0

Work Item:	FS_AIML_CN_Ph2_SEC

Comments

This contribution proposes to update solution#2, addressing the following Editor's Note and adding an evaluation.

Editor's note: How shared key derivation and service operations for exchange of shared key between AMF/SEAF and DCF (option #2) are FFS.

The DCF retrieves a PSK from AMF using 5G-GUTI received from UE in ClientHello message. UE and AMF derive the PSK using SUPI and FQDN/IP of DCF.

* * * First Change * * * *

## 6.3	Solution #2: Security for Data Collection using a DCF

### 6.3.1	Introduction

This solution addresses Key Issue #1.

This solution builds on TR 23.700-04 (for the standardized transfer of standardized data over UP for UE-side data collection) and introduces security enhancements in the 5GS for secure UE connection setup and data transfer with a Data Collection Function (DCF).

### 6.3.2	Solution details

Architecture scope and roles

-	DCF in the MNO domain manages Data Collection Profiles (DCPs) and orchestrates UE data collection and transfer,

Security functions

1)	Authentication and session protection between UE and DCF

-	The UE establishes a secure association with the DCF using shared key derived from network credentials. Transport security (e.g., TLS) is bound to the shared key. Options for shared key derivation are:

-	Option #1: AKMA-based keys (TS 33.535 [4]). DCF acts as a trusted AF, and obtains KAF from the AAnF over SBI.

Editor's note: The role of DCF acting as AF is FFS needs alignment with SA2.

-	Option #2: KAMF derived shared key. DCF obtains the shared key from AMF over SBI.

UE sends its 5G-GUTI to the DCF in the ClientHello message. DCF requests a PSK for the UE from the AMF, providing 5G-GUTI. UE and AMF derive PSK using KAMF  as input KEY, and parameters including SUPI, FQDN/IP address of DCF.

Editor's note: It is FFS whether TLS implementations allow exporting 5G-GUTI in the ClientHello to the SBA layer”.

2)	UE authorization and policy enforcement

-	The DCF authorizes a UE to participate per DCP, using subscription, consent, and operator policy.

Editor's note: Whether and how DCP is applicable is FFS and depends on SA2.

3)	Consent enforcement inside the Core Network

-	The DCF acts as the consent enforcement point for data collection from the UE, i.e., checks consent from UDM/UDR for permissions, as per TS 33.501 [3], Annex V.

Editor's note: Whether and how user consent exposure applies will be decided by SA3 based on SA6 progress.

### 6.3.3	Evaluation

Editor's note: The need for UE authentication is FFS

Editor's note: Further evaluation is FFS.

* * * End of Change * * * *