# 3GPP TSG-SA3 Meeting #125														 draft_S3-254596-r1

# Dallas, US, 17 – 21 November 2025																was S3-254379

Source:	Xiaomi

Title:	Pseudo-CR on New sol for TR 33.700-23 KI#1

Document for:	Approval

Agenda item:	5.2.10

Spec:	3GPP TR 33.700-23

Version:	0.1.0

Work Item:	FS_CAPIF_Ph4_SEC

Comments

This contribution proposes a new solution for TR 33.700-23 KI#1.

* * * First Change * * * *

6.Y	Solution #Y: Client credentials flow based group authorization

6.Y.1	Introduction

This solution addresses KI#1.

This solution uses the client credentials flow to enable the group authorization.

Specifically, the CCF uses the locally stored group related authorization information to authorize the API invoker.

6.Y.2	Solution details

Figure 6.Y.2-1: Client credentials flow based group authorization

It is assumed that the group resource owner has provisioned the group authorization information to the CCF.

1-3. are identical to steps 1-3 defined in clause 8.34.3 of TS 23.222 [2].

4.	The CCF identifies the group authorization information based on the group identifier. The CCF authorizes the API invoker based on the group authorization information.

5.	is similar to step 5 defined in clause 8.34.3 of TS 23.222 [2]. The authorization response includes the token. The token additionally includes the group identifier, which is used to indicate that the token is generated with the group authorization information.

Editor’s Note: The group identifier in the token is FFS.

6-7. are identical to steps 6-7 defined in clause 8.34.3 of TS 23.222 [2].

6.Y.3	Evaluation

Editor’s Note: Evaluation is FFS.

* * * End of Changes * * * *