# 3GPP TSG-SA3 Meeting #125														draft_S3-254611-r1

# Dallas, US, 17 – 21 November 2025

Source:	Ericsson

Title:	KI#2 update

Document for:	Approval

Agenda item:	5.2.7

Spec:	3GPP TR 33.777

Version:	0.2.0

Work Item:	FS_Sensing_SEC

Comments

Proposal to update the KI#2.

* * * First Change * * * *

# 1	Scope

## 5.2	Key Issue #2: Security protection for sensing service operations

### 5.2.1	Key issue details

According to TR 23.700-14 [2], after the sensing service request from the service consumer is authorized by the network, sensing service operations will be triggered and performed by the relevant network functions, which communicate with each other to obtain the sensing result.

In TR 23.700-14 [2], there are multiple solutions proposing sensing service operation procedures supported by sensing entities and different sensing related network functions (e.g. NEF, SF). The NEF needs to discover and select the SF to trigger sensing service operation. The SF needs to select proper sensing entity to collect sensing data in a specific sensing mode. When any of the service conditions of a sensing service is no longer met, an ongoing sensing service can be revoked by the network. The security aspects of all these sensing operations and procedures are to be addressed in this key issue.

NOTE 1:	Security aspects of service operation revocation triggered by sensing function is addressed in this key issue, as it can be viewed as one type of sensing service operations.

### 5.2.2	Security threats

If the connection between sensing entity and sensing function is not securely established, an attacker is able to tamper or inject or replay sensing control messages and sensing data, or sniff the collected sensing data.

### 5.2.3	Potential security requirements

The 5G system shall be able to support integrity protection, confidentiality protection and replay protection for the connection between sensing entity and SF.

Editor’s Note: More security requirements will be added depends on SA2 progress.

* * * End of Changes * * * *