# 3GPP TSG-SA3 Meeting #125																			S3-254218-r2

# Dallas, US, 17 – 21 November 2025

Source:	BSI (DE), Deutsche Telekom

Title:	pCR for 33.521 Clarification of Data Masking

Document for:	Approval

Agenda item:	5.1.3

Spec:	3GPP TS 33.521

Version:	19.0.0

Work Item:	SCAS_5GA

Comments

The test case “Protecting data and information – data masking on integration analysis” is not complete/unclear. This pCR tries to enhance the pre-conditions and test steps in order to make the test case less ambiguous and automatable.
Based on S3-253455 changes from SA3 Meeting #124.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[XX]	3GPP TS 29.552: "Network Data Analytics signalling flows".

* * * Next Change * * * *

##### 4.2.1.2.6	Protecting data and information – Data masking on integration analysis

Requirement Name: Data masking on integration analysis about personal data

Requirement Reference: In accordance with industry best practice.

Requirement Description: NWDAF can collect data from OAM, MDAF and/or 5GC NFs (e.g. AMF) etc. for analytics purposes. Since personal data of the users could be involved, there is a potential privacy impact. As the NWDAF can expose its service operations with a request for bulked data, anonymization of data fields shall be applied to avoid exposing undesired information, aggregation levels.

Threat References: TR 33.926 [4], clause 5.3.6.7, Personal Identification Information Violation

Test Name: TC_DATA_MASKING

Purpose:

Verify that no privacy-related information of the subscribers is disclosed to any entity who is not authorized to access such information.

Pre-Condition:

Privacy information list (contains e.g. PII, location data, network identifiers, session information; should be specified based on local policy, regulation and others).

Preconfigured UE with valid subscriber identifiers and user consent for data collection is granted

The following entities are operational, integrated and simulated:

-	NWDAF

-	‘data producer’ (NF- or OAM as source for data collection which generates user data containing privacy info, e.g. AMF)

-	‘analytics consumer’ (NF- or OAM to which the NWDAF exposes analytics)

The data producer is configured to receive and accept subscription requests from the NWDAF for events according to TS 29.552 [XX], clause 5.5.1.1

Execution Steps:

1.	Tester triggers behaviour so that the ‘data producer’ is required to handle privacy information (e.g. for AMF trigger registration request at UE)

2.	The tester sends an Nnwdaf_AnalyticsInfo_Request request message from the ‘analytics consumer’ to NWDAF according to TS 29.552 [XX], clause 5.2.3.1. The request message shall be crafted to capture information from step 1.

3.		The tester retrieves the Nnwdaf_AnalyticsInfo_Request response message from the NWDAF

Expected Results:

The analytics results do not reveal subscriber permanent identifier nor any other data listed on the Privacy information list.

Expected format of evidence:

Evidence suitable for the interface, e.g. screenshot, pcap trace, log files containing the results.

* * * End of Changes * * * *