# 3GPP TSG-SA3 Meeting #125																	draft_S3-254658-r1

# Dallas, US, 17 – 21 November 2025

Source:	vivo, THALES

Title:	New Key Issue on AEAD Keys

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.1.0

Work Item:	FS_AEAD

Comments

This contribution proposes new Key Issue on AEAD Keys.

The contribution uses noted contribution S3-253788 as baseline. The changes are made based on the discussions in 3GPP-SA3#124 meeting using yellow background.

* * * First Change * * * *

5.X	Key issue #X: AEAD Keys

5.X.1	Key issue details

The AEAD algorithms differ from the current set of algorithms as they use a single key for both encryption and integrity protection. However, the existing key hierarchy does not include single keys to be used by AEAD algorithms.

As described in clause 6.2 of TS 33.501 [5], the existing 5G key hierarchy is derived from a long-term key to NAS, RRC, and UP keys. These keys follow a structure where two separate 128-bit keys are used: one for encryption and the other for integrity protection.

In contrast, AEAD algorithms utilize a single key to perform both encryption and integrity protection.

The key issue is to study how the key derivation needs to be enhanced to cover the key need for the AEAD algorithms.

5.X.2	Security threats

Inappropriate AEAD key derivation can lead to breach of confidentiality or integrity.

5.X.3	Potential security requirements

6GS shall support key derivation for keys for the AEAD algorithms.

NOTE: Full key hierarchy is studied in the scope of TR 33.801-01.

* * * End of Changes * * * *