# 3GPP TSG-SA3 Meeting #125																			draft_S3-254664-r1

# Dallas, US, 17 – 21 November 2025

Source:	vivo

Title:	New Solution on AEAD Algorithm Selection

Document for:	Approval

Agenda item:	5.3.2

Spec:	3GPP TR 33.771

Version:	0.1.0

Work Item:	FS_AEAD

Comments

This contribution proposes a new solution to key issue #1.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 |
| --- | --- | --- | --- | --- | --- |
| Solution Y | X |  |  |  |  |

* * * Next Change * * * *

## 6.Y	Solution Y: AEAD Algorithm negotiation

### 6.Y.1	Introduction

This solution addresses the key issue #1. The solution lists possible AEAD algorithm negotiation for both AEAD-only and AEAD & standalone options.

### 6.Y.2	Solution details

UE sends UE 6G Security capabilities to the network entity.

The network entity select AEAD algorithm based on the UE 6G Security capabilities and algorithm priority list.

AEAD-only:

The UE 6G Security capabilities only include NCA algorithms, i.e. NCA4, NCA5, NCA6.

The selected AEAD algorithm (e.g. NCA4) is indicated to the UE.

AEAD & standalone:

The UE 6G Security capabilities include NIA and NEA algorithms, e.g. NIA4, NEA4, NIA5, NEA5. Once the UE supports both NIA and NEA for the same algorithm, it means that the UE also supports related NCA. For example, if the UE support both NIA4 and NEA4, it implies that UE also supports NCA4.

Editor’s Note:	For case that UE supports both NIA and NEA for the same algorithm, but does not support the related NCA is ffs.

Editor’s Note:	For AEAD & standalone algorithms, whether signaling NCA algorithms as UE security capability is sufficient is ffs.

The selected AEAD algorithm (e.g. NCA4) is indicated to the UE.

For both options:

In case that signalling encryption is not activated (e.g. NAS encryption is not activated), an additional indication is indicated to the UE by the network side along with the selected AEAD algorithm. Upon receiving the indication, the UE uses AEAD algorithm to integrity protect the message only.

### 6.Y.3	Evaluation

TBA.

* * * End of Changes * * * *