3GPP TSG-SA3 Meeting #125	draft_S3-254678-r1

# Dallas Texas, 17-21 November 2025							was S3-254132 merged w/ S3-254115

Source:	Johns Hopkins University APL, IIT Bombay

Title:	pCR on replayed messages on SBA layer

Document for:	Approval

Agenda item:	5.1.1

Spec:	TS 33.502

Version:	0.2.0

Work Item:	SECHAND

Comments

This contribution proposes to add a set of specific information elements for a security related event associated with replayed messages received from one or more originating NFs. It is possible that subsequent replayed message(s) would originate from a different NF from the original message or from multiple NFs in scenarios where an attacker is attempting to gain unauthorized access through impersonation as an example,

Proposed Changes

* * * First Change – All New Text * * * *

# 6.x	Security events related to replayed messages in the SBA layer

The NF collects information on the SBA layer associated with a message being replayed from one or more originating NFs.

Editor’s Note: Whether and how to detect replay messages such as CCA token replay is FFS.

In addition to the common information elements of clause 6.2, this type of events shall include the following:

- Message: The full message which triggered this event.

- Message frequency: The replayed rate of the message.

- NF Consumers: Identifiers of the NFs where the replayed message originated.

Editor’s Note: Any other IEs are FFS.

* * * End of Changes * * * *