3GPP TSG-SA3 Meeting #125	S3-254681

Dallas, USA, 17 – 21 November 2025

# Source:	Vodafone, T-Mobile US, AT&T, China Mobile, Verizon, Charter Communications, Telecom Italia, KDDI, NTT DOCOMO

# Title:	pCR to TS 33.502 – Protection for configuration of events

# Document for:	Approval

Agenda Item:	5.1.1

# 1	Decision/action requested

Agree tdoc for incorporation into the draft TS 33.502

# 2	References

[1]	3GPP TS 33.502 v0.2.0

# 3	Rationale

Based on the requirements for security related events, the configuration of the NFs for detection of the events needs to be secured. This proposal specifies the protection to be applied to the interface used to configure the NFs for the events to be detected.

# 4	Detailed proposal

* * * First Change * * * *

# 7	Protection of Security related events

Editor’s Note: This clause addresses the protection for the configuration, collection and delivery of events.

## 7.x	Protection for the configuration and enabling/disabling detection of security related events

TLS shall be supported and used to provide mutual authentication, integrity protection, replay protection and confidentiality protection for the interface between the Events transmitter and the Management Entity handling the configuration and the enabling/disabling of events collection.

NOTE 1: If the interface between the Events transmitter and the Management Entity is trusted (e.g. physically protected), it is for the PLMN-operator to decide whether to use cryptographic protection.

Security profiles for TLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks.

NOTE 2: A PLMN-operator policy can use dedicated certificates for this secure communication.

* * * End of Changes * * * *