3GPP TSG-SA3 Meeting #125	draft_S3-254699-r2

# Dallas, USA, 17 – 21 November 2025

Source:	vivo

Title:	New solution: privacy protection

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.1.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

This contribution proposes a new solution on ID privacy for TR 33.714.

* * * First Change * * * *

## 5.1	Mapping of solutions to key issues

Editor’s Note: This clause captures mapping between key issues and solutions.

Table 5.1-1: Mapping of solutions to key issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | #4 |  |
| Y | X |  |

* * * Second Change* * * *

## 5.Y	Solution #Y: ID privacy based on stored type T-ID

### 5.Y.1	Introduction

This solution addresses KI#4.

This solution proposes to reuse T-ID update method of release 19 as much as possible. In release 19, two T-ID types are defined: concealed and stored. This solution propose to reuse stored type T-ID and corresponding update method.

### 5.Y.2	Solution details

The following figure depicts the AIoT device ID protection based on stored type T-ID.

Figure 5.Y.2-1: AIoT Device ID protection based on stored type T-ID

1.	AIoT device is preconfigured with initial T-ID (i.e. T-ID0), which can be derived from AIoT device permanent ID. The ADM also stores the initial T-ID for the AIoT device. After some registration procedures, the initial T-ID has been updated to T-IDn.

Editor’s Note:	How to pre-configure the initial T-ID is ffs.

Editor’s Note:	How the solution applies to the T-ID variants as defined in TS 33.369 is FFS.

2.	AIoT device sends initial Register Request (T-IDn) towards AIOTF.

3.	AIoTF sends Authentication Request (T-IDn) towards ADM.

4.	AIoT device, AIOTF and ADM performs authentication procedure.

5-7.	If authentication succeeds, AIOTF retrieves new T-ID from ADM as described in step 10 of clause 5.4.3 in TS 33.369 [7].

8.	As described in steps 2 and 3 of clause 5.4.3 in TS 33.369 [7] with exception that the AIOTF sends Register Response with the T-ID handling information to AIoT device.

Editor’s Note:	Update on usage of the same T-ID between registration is ffs.

9.	As described in "after step 10" of clause 5.4.3 in TS 33.369 [7].

The T-IDn+1 is used for successive initial registration procedure.

### 5.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

Editor’s Note:	Evaluation of T-ID storage on device side in de-registration state is ffs.

TBD

* * * End of Changes * * * *