3GPP TSG-SA3 Meeting #125	S3-254706-r2

# Dallas, US, 17 – 21 November 2025

Source:	Nokia, OPPO, CATT, Samsung, Xiaomi

Title:	KI1: security threats and requirements

Document for:	Approval

Agenda item:	5.2.11

Spec:	3GPP TR 33.714

Version:	0.1.0

Work Item:	FS_AIoT_SEC_Ph2

Comments

This pCR proposes a security threat and requirement to the KI on intermediate UE authorization. The threat and requirement are unchanged compared to release 19, as the KI was discontinued by the plenary and is resumed in release 20.

This is a merger of S3-254422, S3-254368, S3-254453, S3-254464 and S3-254471.

* * * First Change * * * *

## 4.1	Key Issue #1: Authorization of intermediate UE for 5G Ambient IoT services

### 4.1.1	Key issue details

In TR 23.700-13 [2], Key Issues #1 and #3 describe the issues on the system architecture and procedure to support 5G Ambient IoT services, furthermore TR 23.700-30 [4], KI#1 describes the issues on the support AIoT services under the RRC-based option for UE Reader connectivity.

The architecture for topology 2 is defined in TR 23.700-13 [2] clause 8.1.3 which forms the baseline for the release 20.

In the Topology 2 as defined in TR 38.848 [3], the UE is acting as the intermediate node responsible for transferring the information between AIoT device and 5GS. If the authorization and authentication of the intermediate node is not supported, the attacker can play the role of an intermediate node and arbitrarily deny 5G AIoT service to the AIoT device.

Therefore, it is necessary to study how to authorize a UE for acting as the intermediate node i.e an AIoT reader.

NOTE: According to TR 38.769 [5], the intermediate UE for Device 1, 2b and C are all in the scope of this issue.

### 4.1.2	Security threats

If the 5GC do not authorize  the UE acting as an intermediate node, the attacker UE may misuse the Ambient IoT services provided by the core and hereby impersonate an authorised intermediate node. The attacker UE may then deny access to 5G Ambient IoT services.

Editor’s Note: The threats may be refined based on SA2 agreed procedures.

### 4.1.3	Potential security requirements

The 5GS shall be able to support the authorization of the AIoT capable UE as an intermediate node.

Editor’s Note: Requirements are FFS.

* * * End of Changes * * * *