3GPP TSG-SA WG4 Meeting #134	S4-251696

Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Summary of the MPEG progress on 3DGS

Agenda item:	9.7

## 1. Introduction

Gaussian splatting [1] is a recently developed immersive representation that provides photorealistic visual quality for novel view synthesis (with 6 degrees of freedom or 6DoF). Compared to other immersive representations such as meshes, multi-view video  or point clouds, this representation can handle complex visual content including reflective (non-Lambertian) surfaces or volumetric effects (such as fire, smoke etc.) with high visual quality. This representation is typically learned from single or multiple cameras inputs and yields compelling results with relatively fast training times. Furthermore, Gaussian splatting also allows for real-time rendering with photorealistic visual quality. However, the representation data size is large, and there is a need for efficient compression technologies for Gaussian splatting. For all these reasons, MPEG has recently started an exploration activity on Gaussian splat coding (GSC).

This contribution summarizes MPEG’s relevant work to date, providing references to the related drafts and output documents.

## 2. MPEG progress on 3DGS

- Use cases and requirements (public output document)

MPEG SC 29 WG 2 produced two output documents on Draft Gaussian splat coding requirements [2] and Draft Gaussian splat coding use cases [3]. A list and description of 22 use cases for Gaussian splat coding has been collected in [3] and provides a classification of the use cases based on how the content is captured and whether it is static or dynamic. Requirements are provided in [2], detailing representation requirements and coding requirements, respectively. Most coding requirements are common to all use cases, while a subset of requirements is use-case-specific as highlighted in [2]. Requirements also define two paradigms to evaluate Gaussian splat coding; I-3DGS and A-3DGS. (see Figure 1). I-3DGS focuses on the Inria representation [1] and processes Gaussian splat models as input of the encoder without access to capture views. This is similar to the Niantic SPZ codec paradigm. A-3DGS, on the contrary, starts from captured views, allows defining other Gaussian splat model representations than the Inria one [1]. MPEG currently focuses on the I-3DGS track, considered closer to market needs and readiness than A-3DGS, which is also explored to follow and adapt to research advances in the field.

- The Lightweight GSC specific case (public output document)

MPEG also identified that there is an urgent need for standardized solutions, ensuring operability, for the specific case of mobile devices, which require more lightweight codecs and content. This led to the definition of lightweight GSC requirements [4], focusing on static Gaussian splat content and mobile device today's hardware for capturing, learning, encoding and decoding Gaussian splats on mobile phones. In order to answer the need for fast time to market solutions, so-called fast tracks (fast standardization tracks) were defined to explore coding tools able to reach the complexity requirements for lightweight GSC on the one hand, and to meet the fast standardization timeline (~1 year) on the other hand.

- V-PCC Amendment and its timeline (public output document)

Out of the fast tracks focusing on static content, lightweight complexity solutions and short standardization timeline (I-3DGS paradigm), a solution based on a V-PCC Raw Profile, using only a subset of V-PCC specifications and coding tools, i.e., raw patches, has emerged. An Amendment of V-PCC has been initiated [5 , 6, 7] to enable Gaussian splat coding. Instead of using projections, the raw patches allow to segment the Gaussian splat model into a set of 3D boxes (similarly to a k-d tree) and to rasterize splats as points in raw patches that are packed in video frames. As V-PCC relies on video codecs that have mobile hardware support, it leads to significant compression gains when compared to SPZ, or Self-Organized Maps approaches (the MPEG GSC video anchor).

The timeline spans one year for this V-PCC Amendment as follows: CDAM on 2026-01-29, DAM on 2026-04-29, FDAM on 2026-07-28 and AMD on 2026-10-31.

The V-PCC Amendment will be the first MPEG standard supporting GSC by the end of 2026.

- Ongoing explorations and a note on Dynamic GSC

Besides working on the V-PCC Amendment for Gaussian splat coding, MPEG continues exploring several approaches such as Geometry-based coding of Gaussian splats as well as other video-based approaches, for example using VSEI messages.

Dynamic Gaussian splat coding is seen as important but more longer-term, requiring input sequences, preferably with tracked or partially tracked Gaussian splats (Gaussian splats from the first frame are animated for the next frames, allowing reducing flickering and stabilizing the number of splats through time). More content is needed to explore further this interesting avenue.

- Common Test Conditions

MPEG also released publicly its Gaussian splat coding Common Test Conditions [8], providing information on its reference tools for preprocessing and postprocessing Gaussian splat models, computing metrics, rendering Gaussian splats and listing all available content for evaluation.

## 3. References

[1]	Bernhard Kerbl, Georgios Kopanas, Thomas Leimkühler, and George Drettakis. 2023. 3D Gaussian Splatting for Real-Time Radiance Field Rendering. ACM Transactions on Graphics 42, 4 (July 2023). https://repo-sam.inria.fr/fungraph/3d-gaussian-splatting/

[2]	ISO/IEC JTC 1/SC 29/WG 2 N00487, Draft Gaussian splat coding requirements, Geneva, October 2025.

[3]	ISO/IEC JTC 1/SC 29/WG 2 N00486, Draft Gaussian splat coding use cases, Geneva, October 2025.

[4]	ISO/IEC JTC 1/SC 29/WG 2 N00485, Preliminary draft requirements for lightweight GSC, Geneva, October 2025.

[5]	ISO/IEC JTC 1/SC 29/WG 7 N01300, Request for ISO/IEC 23090-5 Amd.1: V-PCC for Gaussian splats coding, Geneva, October 2025.

[6]	ISO/IEC JTC 1/SC 29/WG 7 N01302, Working draft of V-PCC Amd. for Gaussian splats coding, Geneva, October 2025.

[7]	ISO/IEC JTC 1/SC 29/WG 7 N01343, Description of CEs to support Gaussian splats coding in V-PCC, Geneva, October 2025.

[8]	ISO/IEC JTC 1/SC 29/WG 4 N00751, Common test conditions for Gaussian splat coding, Geneva, October 2025.

## 4. Proposal

The current document is provided to inform the group about relevant work in MPEG.

The text in clause 1, 2, 3 and annex can be included to the technical report or permanent document as relevant work in MPEG.

## Annex: Figures

Figure 1: MPEG GSC paradigms. I-3DGS where the input format of the encoder is a Gaussian splat model frame, based on the Inria representation [1], with no access to ground truth views. The output of the decoder is the same representation. Metrics computations are only done in image space by first rendering both the input uncompressed Gaussian splat model and the output reconstructed Gaussian splat model and finally by comparing those rendered images with image metrics. A-3DGS, is another paradigm where the input representation consists of the captured views. The learned Gaussian splat model can be different from the Inria representation [1]. In this case, metrics are computed in image space, comparing ground truth input views and output reconstructed and rendered views.

Figure 2: Draft GSC use cases [3] classified based on their capture type and their temporal nature (static or dynamic)