3GPP TSG-SA WG4 Meeting #134	S4-251906

# Dallas, Texas, USA, 17 – 21 November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 13 | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification to AssociatedInfo and related changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Canada |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED, TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The associated information and corresponding API, data model, is unconsistent with the call flow. Some fields in the associated information are underdefined, while being mandatory.There is a need to clarify that the associated information are used with an avatar representation (to enable selecting a representation) and not with an asset. Corresponding changes in the call flow are in CR S4-251901 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the cardinality of the avatarMetadata object - Make the associated information API applicable to the Avatar Representation, not to the base avatar model, and not to the asset. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is unclear, and could lead to non-interoperable implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.5, B.1.6, B.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Introduction

The specification includes an Associated Information API in clause B.1. 7. However, it is unclear whether the associated information are linked to the Avatar Representation or to the base avatar model, or if it should be associated to both. Moreover, the Associated Information seems to also be linked to the Asset resource in the Assets API. The purpose of the associated information API needs to be clarified.

In the management call flow, the two following steps indicate that the associated information are linked to the Avatar ID:

6.3	The BAR creates a new Asset ID for the new asset and associates it to the Avatar ID of the base avatar model.

6.4	The BAR updates the associated information corresponding to the Avatar ID.

It seems missleading to associate these information to the Asset ID as currently done in clause B.1.6

In addition, our reading of the associated information is that it applies to the Avatar Representation, and not to the ARF container / base avatar model as per the selectionInfo:  “Provides information that the user can use to select this avatar. This may contain a name, a nickname of the asset, usage context e.g. casual, work, and images of renditions of the asset.”

Futher, the Associated Information table includes an avatarMetadata object, which is not used in the rest of the specification and is undefined.

The proposed changes are as follow:

- Change the cardinality of the avatarMetadata object

- Make the Associated Information API applicable to the Avatar Representation, not to the base avatar model, and not to the asset.

  - An alternative would be to apply the associated information to the base avatar model and to the avatar representation (but not to the asset).

# 2	Poposed Changes

* * * First Change * * *

## B.1.5	Avatars API

### B.1.5.1	Overview

The Avatars API is used by the DC AS or MF to manage Base Avatar (including related assets) in the BAR, providing operational functions such as Base Avatar creation, retrieval, update and deletion.

### B.1.5.2	Resource structure

The Avatars API is accessible through the following URL base path:

{apiRoot}/3gpp-mbar-management/{apiVersion}/avatars/

Table B.1-4 specifies the operations and the corrresopnding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table B.1-4: Operations supported by the Avatars API

| Operation name | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Avatar |  | POST | Creates a new base avatar model resource in the BAR. |
| Get Avatar | {avatarId} | GET | Used to retrieve a previously created or uploaded base avatar model in the BAR. |
| Update Avatar |  | PUT, PATCH | Used to upload or update Base Avatar model data corresponding to an Avatar ID. |
| Delete Avatar |  | DELETE | Removes and deletes a Base Avatar model, as well as its related assets. |

### B.1.5.3	Data model

#### B.1.5.3.1	Avatar resource

Table B.1-5: Definition of Avatar resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| avatarId | ResourceId | 1..1 | C: RO R: RW U: – | A unique identifier assigned to a Base Avatar model by the BAR on creation. |
| ownerId | string | 1..1 | C: RW R: RO U: – | A unique identifier identifying the subscriber (owner) associated with the base avatar model specified by avatarId in this resource. |
| assetIds | array(ResourceId) | 0..1 | C: RO R: RO U: – | A list of assets associated with the Base Avatar model. |
| avatarContainer | URL | 0..1 | C: RW R: RO U: RW | Payload containing the Base Avatar Model data and associated assets. This provides access to the full binary avatar container, including all of the contained assets. For creation and update operations, the URL shall point to a multi-part mime part with MIME type “model/vnd.mpeg.arf+zip”. |

* * * Second Change * * *

### B.1.6.3	Data model

#### B.1.6.3.1	Asset resource

Table B.1-7: Definition of Asset resource

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| assetId | ResourceId | 1..1 | C: RO R: RW U: – | A unique identifier assigned to an asset by the BAR on creation. The assetId is scoped by the avatarId. |
| namespace | string | 1..1 | C: RW R: RO U: RW | A namespace defining the intended usage of the asset, as exemplified by names such as "human/head" or "accessory/hat" |
| LoD | array(string) | 0..1 | C: RW R: RO U: RW | A list of available LoDs for the corresponding asset. NOTE: The labels for LoDs are FFS. |
| assetData | array(URL) | 0..1 | C: RW R: RO U: RW | List of URLs that point to the asset data. The primary URL shall point into an ARF document that describes all components of the asset. For creation/update of an asset, all components shall be provided as part of a multi-part mime body. |

* * * Third Change * * *

## B.1.7	Associated Information API

### B.1.7.1	Overview

The Associated Information API is used by the DC AS or MF to fetch Associated Information related to an Avatar Representation from the BAR.

### B.1.7.2	Resource structure

The Associated Information API is accessible through the following URL base path:

{apiRoot}/3gpp-mbar-management/{apiVersion}/avatars/{baseAvatarModelId}/representations/{avatarid}associatedInfo/

Table B.1-8 specifies the operations and the corrresopnding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table B.1-8: Operations supported by the Associated Information API

| Operation name | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Get Associated Information |  | GET | Used to retrieve associated information corresponding to an Avatar Representation (identified by the AvatarID) in the BAR. |

### B.1.7.3	Data model

#### B.1.7.3.1	Associated information resource

Table B.1-9: Definition of Associated Information resource

| Property name |  |  | Data type |  | Cardinality |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| associatedInfo |  |  | Object |  | 1..1 |  | A list of assets associated with the Avatar Representation. |
|  | avatarId |  | ResourceId | 1..1 |  | A unique identifier assigned to an Avatar representation by the BAR on creation. |  |
|  | avatarMetadata |  | Object | 0..1 |  | Metadata related to the Avatar Representation, details are FFS. |  |
|  | selectionInfo |  | Object | 0..1 |  | Provides information that the user can use to select this avatar representation. This may contain a name, a nickname, usage context (e.g. casual, work), and images of renditions of the avatar representation. |  |
|  | assetIds |  | array(ResourceId) | 1..1 |  | A list of assets associated with the Avatar representation. |  |
|  |  | assetLoDs | array(Object) | 1..1 |  | A list of available LoDs for the corresponding asset. The resulting size in bytes shall be associated with each LoD. NOTE 1: LoDs descriptions/labels and their associated complexity is FFS. NOTE 2: Clarification about how LoD would be described and what that would mean in terms of complexity is FFS. |  |
|  | supportedAnimations |  | array(string) | 1..1 |  | A list of the URNs that identify the supported animation frameworks by this avatar representation. |  |
|  | infoUpdatedAt |  | number | 1..1 |  | A timestamp (in wall clock time) describing the time of the last update to the associated information for the corresponding Avatar Representation. This field is updated whenever an asset is modified by the owner of the Base Avatar Model or BAR. Users may utilize the infoUpdatedAt field to verify the latest validity of previously downloaded Avatar data. Comparing the infoUpdatedAt value with the downloaded time allows users to determine if the downloaded data requires updating, |  |

* * * Fourth Change* * *

### B.1.8.3	Data model

#### B.1.8.3.1	Avatar representation resource

Table B.1-11: Definition of Avatar representation resource

| Property name |  |  | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- | --- | --- |
| avatarId |  |  | ResourceId | 1..1 | C: RO R: RW U: – | A unique identifier assigned to an Avatar Representation by the BAR on creation. |
| ownerId |  |  | string | 1..1 | C: RW R: RO U: – | A unique identifier identifying the subscriber (owner) associated with the base avatar specified in this resource. |
| associatedInfo |  | AssociatedInfo | 1..1 | C: RO R: RO U: RO | Associated information related to the Base Avatar Model. |
| publishTime |  |  | number | 1..1 | C: RO R: RO U: – | Describes the issue time or latest update time (in wall clock time) of the Avatar Representation. |

* * * End of Changes* * *